# FLH Standard Practice Checklist

Project:  Project Name and Number

Prepared By:  Lead Designer 		Date:

Checked By:  Peer Reviewer 		Date:

Approved By:  Internal:HDM -- A/E:Design Mgr or PM 		Date:

| PDDM Section | FLH Standard Practice | Yes | No | N/A | Remarks |
| --- | --- | --- | --- | --- | --- |
| 9.3.1.11 | Does the minimum superelevation rate (reverse crown) equal normal crown rate? |  |  |  |  |
| 9.3.2 | Have the general design considerations found in the Green Book Chapter 3 §Combinations of Horizontal and Vertical Alignment been applied? |  |  |  |  |
| 9.3.5 | Has the correct CFLHD default superelevation preferences file been used? (if the default preference file has been adjusted by the user, refer to PDDM 9.3.5 to verify that project meets FLH policy and standard practice) |  |  |  |  |
| 9.3.5.2.1 | Does the superelevation runoff meet the following: - On the curve: up to 50% of the runoff length located on the curve - On the tangent: minimum desired = 60% of the runoff length located on the tangent; maximum desired = 80% of the runoff length located on the tangent (90% acceptable in certain situations; refer to PDDM 9.3.5.2.1) |  |  |  |  |
| 9.3.5.2.2 | Have the reverse curve transitions been designed according to PDDM 9.3.5.2.2? |  |  |  |  |
| 9.3.5.2.3 | For compound curve transitions: |  |  |  |  |
|  | Is the relative gradient in this range: - Maximum = the maximum relative gradient value shown in the Green Book Exhibit 3-30 - Minimum = 70% of the maximum relative gradient value shown in the Green Book Exhibit 3-30 |  |  |  |  |
|  | Has the transition length been designed according to PDDM 9.3.5.2.3? |  |  |  |  |
| 9.3.5.2.4 | For broken back curve transitions: |  |  |  |  |
|  | Is the minimum length of normal crown between broken back curves 60 m (200 ft)? |  |  |  |  |

| PDDM Section | FLH Standard Practice | Yes | No | N/A | Remarks |
| --- | --- | --- | --- | --- | --- |
| 9.3.5.2.7 | For spiral curve transitions: |  |  |  |  |
|  | Is the minimum length of spiral curve transition the length required for superelevation runoff? |  |  |  |  |
|  | Is the superelevation runoff applied uniformly over the full length of the spiral? |  |  |  |  |
| 9.3.6.1 | For mainline roadways with design speeds less than 60 km/h (40 mph): |  |  |  |  |
|  | Is the minimum vertical curve length 60 m (200 ft) for grade differences less than 3%? |  |  |  |  |
|  | Is the minimum vertical curve length 100 m (300 ft) for grade differences greater than or equal to 3%? |  |  |  |  |
| 9.3.6.1 | For mainline roadways with design speeds of 60 km/h (40 mph) or higher: |  |  |  |  |
|  | Is the minimum vertical curve length 90 m (300 ft) for grade differences less than 3%? |  |  |  |  |
|  | Is the minimum vertical curve length 120 m (400 ft) for grade differences greater than or equal to 3%? |  |  |  |  |
| 9.3.6.2 | Is the maximum sustained grade 9% or less for the following types of roadways: -potential winter snow-packed conditions or -aggregate surfaces |  |  |  |  |
| 9.3.6.3 | For roadways with level longitudinal grade (0%): -Is there enough crown to drain the roadway along through-fill sections with no curb? -Are there enough crown and special ditch grades to convey drainage in cuts with no curb? |  |  |  |  |
|  | Is the minimum longitudinal grade 0.5% in areas that do not meet the above criteria? |  |  |  |  |
| 9.3.6.3 | Are longitudinal grades more than 1% where superelevation transitions create a pavement edge profile grade less than 0.2% (0.5% for curbed sections)? |  |  |  |  |
| 9.3.6.8 | Does the roadway elevation exceed the headwater elevation near drainage structure inlets? |  |  |  |  |
| 9.3.6.8 | Is the low point of subgrade at least 0.5 m (1.7 ft) above ordinary high-water elevation in swampy terrain and areas subject to flooding? |  |  |  |  |
| 9.3.6.8 | Is there adequate ditch drainage in areas with horizontal curves and grades less than 2%? |  |  |  |  |
| 9.3.7.5 | Has the intersection sight distance been designed according to the Green Book Chapter 9 §Intersection Sight Distance? |  |  |  |  |
| 9.3.9.1 | Does the traveled way curve widening meet Green Book? |  |  |  |  |
| 9.3.9.8.1 | Are the passing zones designed to meet Green Book? |  |  |  |  |
| 9.3.9.8.2 | Are the passing lanes designed according to the PDDM 9.3.9.8.2? |  |  |  |  |
| 9.3.11.3 | Do curb offsets meet the following: |  |  |  |  |
|  | Are vertical curbs offset at least 0.6 m (2 ft) from the normal edge of shoulder? |  |  |  |  |
|  | Are sloping curbs offset at least 0.3 m (1 ft) from the normal edge of shoulder? |  |  |  |  |
| 9.3.11.3 | For roadways without shoulders, do the curb offsets meet the requirements in Exhibit 9.3-L of the PDDM? |  |  |  |  |
| 9.3.14.6 | Left-turn Lanes: |  |  |  |  |
|  | Does the minimum length of acceleration and deceleration lanes meet Green Book? |  |  |  |  |
|  | Does the taper length meet Green Book? |  |  |  |  |
| 9.3.14.7 | Right-turn Lanes: |  |  |  |  |
|  | Does the minimum length of acceleration and deceleration lanes meet Green Book? |  |  |  |  |
|  | Does the taper length meet Green Book? |  |  |  |  |
|  | Do the corner radii accommodate the design vehicle (see PDDM 9.3.14.7)? |  |  |  |  |
| 9.3.14.9 | Is the design speed 70 km/h [45 mph] or less where curbing is used for channelization? |  |  |  |  |
| 9.3.16.1 | Does the sidewalk design accommodate persons with disabilities? |  |  |  |  |
| 9.3.16.1 | Are sidewalks provided in urban areas where there is a need for pedestrian access to schools, parks, commercial areas, or transit stops? |  |  |  |  |
| 9.4.3 | For 3R projects: Has the normal crown on tangent sections been restored to at least 1.5% (2% preferable)? |  |  |  |  |
| 9.4.5 | For 3R projects: Are substandard or deficient signs or pavement markings corrected? |  |  |  |  |
| 9.4.5 | For 3R projects: Are edge line pavement markings used? |  |  |  |  |
| 9.5.1.1 | Do clearing widths extend a minimum of 1.5 m (5 ft) beyond the outer limit of slope rounding for cuts and 1.5 m (5 ft) beyond the toe of fill? |  |  |  |  |
| 9.6.5.3.14 | Do government-designated material sources have a reclamation plan and applicable environmental documentation included? |  |  |  |  |

CFLHD Version 1