FLHBO BRIDGE PLAN CHECKLIST Project Name & Number: _________________________________ Project Designer: _________________________ Team Leader: ____________________________ Checker: ____________________________ Date: ____________

| I GENERAL | Checker Initials |
| --- | --- |
| #### Follow these general detailing practices on all structural drawings as practical: |  |
| Dimension elements once on a drawing, unless repeating is necessary for clarity |  |
| Identify reference lines and use consistently throughout plans. Reference lines include but are not limited to CL Rdwy., CL Bridge, Layout Line, Begin and End Bridge (defined as fill face of endwall or breastwall), CL Bearing, CL Pier |  |
| Scale all drawings or note “Not to Scale” if applicable. Use the appropriate scale bar cell when specifying the scale on the sheet border, and for those details on the sheet at different scales call out the scale. |  |
| Locate all exposed concrete faces and elements in relation to reference lines |  |
| Provide sections and details at as many locations as required to clearly define the intended design, with economy of details in mind |  |
| Provide separate drawings for detailing concrete dimensions and reinforcement layout for complex concrete shapes |  |
| Arrowheads and section arrows are same size and style regardless of drawing scale |  |
| Text font and size are consistent |  |
| Use standard centerline symbol as produced by key function |  |
| Order of PS&E bridge drawings follows the general order outlined in Section III (General Notes may appear on General Plan and Elevation sheet if there is sufficient room) |  |
| Locate all reinforcement from exposed concrete faces |  |
| Identify all reinforcement in two separate views |  |
| Identify all bars that are to be epoxy coated with “E” (i.e. #4EA bars) |  |
| Identify reinforcement clearances if other than 2” |  |
| Identify all minimum lap splice lengths |  |
| Refer to concrete faces as “fill face”, “other face”, or “each face” or respective abbreviations “f.f.”, “o.f.”, “e.f.”. Use standard cell for abbreviation table. |  |
| Detail bar placements following the convention “x spa. @ x = xx”. Try to avoid designation “x equal spa. = x”, for construction considerations. |  |
| When dimensioning to centerlines, solidify that section of line to be dimensioned to |  |
| Arrows used at ends of leader lines for pointing to specific points or surface lines. Squiggles (~) at ends of lines used for specifying surface areas. |  |
| Project block (upper right hand corner) shall contain the project name, and sheet numbering (as established by Project Development) |  |
| Information entered in the border blocks at the bottom of sheet should include sheet title name, structure number (inspection report no. for NPS), structure name, featured intersection, location (county, state, partner name), names, dates, revisions, scale, date, RG number (CFLHD and WFLHD), etc. Names spelled out unless abbreviations are necessary because of length. |  |
| Include NPS PMIS No. and Drawing No. under sheet title (if applicable) |  |
| “PRELIMINARY NOT FOR CONSTRUCTION” label removed from all sheets for Advertisement Plan package |  |
| II PRELIMINARY ENGINEERING BRIDGE PLANS – TYPE, SIZE AND LOCATION | Checker Initials |
| #### A) PLAN |  |
| Existing topography (enough contours for a sufficient reach of stream to indicate streamflow alignment) |  |
| Plan view of structure including walls, approach slabs, railings (including transitions) |  |
| Outline (ghost line) of existing structure (if applicable) |  |
| North arrow |  |
| Transverse width of traveled ways and shoulders of roadways below structure |  |
| Skew angle (if applicable) |  |
| Limits of slope paving or toe of fill (if applicable) |  |
| Stream name and direction of flow or traffic direction below structure |  |
| Estimated wingwall lengths and orientation |  |
| Identify Sta. for intersection of CL Rdwy. of structure and CL Rdwy./RR below structure (if applicable) |  |
| Utilities on and off bridge (if applicable) |  |
| Horizontal Curve Data (if applicable) |  |
| Horizontal PC and PT stations and other significant stations |  |
| #### B) ELEVATION |  |
| Elevation view of structure including railings, substructure, walls, slope protection, and aesthetic treatments |  |
| Station and elevation of Begin and End bridge and CL piers |  |
| Length of bridge and span lengths |  |
| Label railing types (include pedestrian screen), superstructure type, slope protection type, foundation type, aesthetic treatments (as known) |  |
| Elevation bar |  |
| Minimum vertical clearance requirements and horizontal and vertical construction clearance requirements for bridges over Rdwys./RR (if applicable) |  |
| Utilities off structure if known (if applicable) |  |
| Existing ground line at CL Rdwy., and at left and right edges of deck if topography varies significantly |  |
| CL Rdwy./RR below structure (if applicable) |  |
| Design discharge and water surface elevation, and freeboard provided (at low point) |  |
| External access doors located (if applicable) |  |
| C) ROADWAY |  |
| Typical roadway section at bridge |  |
| Superelevation data (if applicable) |  |
| Profile grade diagram (verify sag point of vertical curve not on structure) |  |
| D) HYDRAULICS |  |
| Hydraulic Data Table including discharges, velocities, and water surface elevations for 2, 50, 100, 200 (or 500) year events and freeboard requirements (as provided) |  |
| Slope protection and scour requirements (if available) |  |
| Hydraulics Report No. (if available) |  |
| II PRELIMINARY ENGINEERING BRIDGE PLANS – TYPE, SIZE AND LOCATION cont’d. | Checker Initials |
| E) TYPICAL SECTION |  |
| Split section with abutment/pier section and midspan section |  |
| Transverse width of traveled way, railings, sidewalks on structure |  |
| Show all girders, slab, railings, sidewalks, wearing surface, drains, utilities |  |
| Cross slopes including sidewalks |  |
| Railing height, superstructure depths |  |
| F) DESIGN DATA |  |
| Live loads, overloads, special loads |  |
| Superstructure type, railing and transition types |  |
| Concrete strengths, epoxy reinforcement requirements |  |
| Special design requirements |  |
| G) PROJECT DATA |  |
| Bridge name, route name |  |
| Structure Number (supplied by BIP if applicable) |  |
| Stream or route name of crossing feature |  |
| Project location (state and county) |  |
| Project Number |  |
| Owner government agency |  |
| Estimated construction cost |  |
| Scale |  |
| Date |  |
| H) MISCELLANEOUS |  |
| Construction sequence details (if applicable) |  |
| Aesthetic treatments (if applicable) |  |
| Materials Report Number (if available) |  |
| III PS&E BRIDGE PLANS – 1) GENERAL PLAN AND ELEVATION | Checker Initials |
| #### A) PLAN |  |
| Plan view of bridge including walls, approach slabs, expansion joints, railings (including transitions) |  |
| North arrow |  |
| Station and elevation of Begin and End bridge and CL piers |  |
| Length of Bridge and span lengths |  |
| Transverse width of traveled way, railings, and sidewalks on structure |  |
| Transverse width of traveled ways and shoulders of roadways below structure |  |
| Skew angle (if applicable) |  |
| Limits of slope paving (if applicable) |  |
| Deck drain/scupper locations (if applicable) |  |
| Identify points of minimum vertical clearance (if applicable) |  |
| Traffic direction below structure (if applicable) |  |
| Label wingwalls if different geometry (i.e. “WINGWALL A”, “WINGWALL B”, etc.) |  |
| Identify Sta. for intersection of CL Rdwy. of structure and CL Rdwy./RR below structure (if applicable) |  |
| Utilities on and off bridge (if applicable) |  |
| Horizontal PC and PT stations and other significant stations |  |
| Sign and light standard locations (if applicable) |  |
| #### B) ELEVATION |  |
| Elevation view of bridge including railings, substructure, walls, slope protection, and aesthetic treatments |  |
| Label railing types (include pedestrian screen), superstructure type, slope protection type, aesthetic treatments |  |
| Elevation bar |  |
| Identify minimum provided vertical clearances and horizontal and vertical construction clearances for bridges over Rdwys./RR (if applicable) |  |
| Utilities off structure if known (if applicable) |  |
| Existing ground line at CL Rdwy., and at left and right edges of deck if topography varies significantly |  |
| CL Rdwy./RR below structure (if applicable) |  |
| Design discharge and water surface elevation |  |
| Label all bearings as FIX for fixed or EXP for expansion bearings |  |
| Label external access door locations (if applicable) |  |
| C) MISCELLANEOUS |  |
| Profile Grade Diagram (if any part of structure on Vert. Curve) |  |
| Horizontal Curve Data (if applicable) |  |
| Hydraulic Data Table including discharges, velocities, and water surface elevations for 2, 50, 100, 200 (or 500) year events and freeboard requirements (as provided) |  |
| Superelevation Diagram (if applicable) |  |
| Index of Drawings |  |
| III PS&E BRIDGE PLANS – 2) GENERAL NOTES AND ESTIMATE | Checker Initials |
| #### A) GENERAL NOTES |  |
| Specifications (Design and Construction) |  |
| Design Loading |  |
| Design Criteria |  |
| Seismic Design Criteria (if applicable) |  |
| Concrete |  |
| Structural Steel |  |
| Reinforcing Steel |  |
| Prestressed Concrete Members |  |
| Finishing Concrete |  |
| Removal of Structures and Obstructions |  |
| Utilities |  |
| Existing Bridge Plans (if available) |  |
| B) ESTIMATE (CFL AND WFL only) |  |
| Table with contract item #, contract item, and quantity |  |
| Notes to describe payment for subsidiary items, and identify contract pay items |  |
| C) MISCELLANEOUS |  |
| Non-standard vehicle diagrams showing axle spacings and loads (if applicable) |  |
| Scope of Work (for rehabilitation projects) |  |
| III PS&E BRIDGE PLANS – 3) FOUNDATION LAYOUT | Checker Initials |
| #### A) PLAN |  |
| North arrow |  |
| Label stations at reference lines |  |
| Pile/shaft sizes/ type |  |
| Geometry to locate piles/shafts and or footings |  |
| Indicate batter piles |  |
| Stage construction details (if applicable) |  |
| Existing contours |  |
| Stream name, direction of flow, and edge of stream |  |
| Outline (ghostline) of existing structures (if applicable) |  |
| Temporary shoring and bracing limits (if applicable) |  |
| Limits of slope protection and associated details (if complex shown on separate sheet) |  |
| Centerline bearing station |  |
| B) ELEVATION |  |
| Foundation with cap/footing bottom elevation, and embedment depths |  |
| Existing groundline |  |
| Estimated tip elevation and minimum tip elevation (if scour) |  |
| Indicate pile batter |  |
| Slope protection |  |
| Foundation size and type |  |
| C) MISCELLANEOUS |  |
| Foundation Notes |  |
| Foundation Data Table with location, nominal driving resistance, maximum factored axial load, and length of piles/shafts (or estimated tip and minimum tip elevations) |  |
| Pile tip details (if applicable) |  |
| D) PRESTRESSED PILES |  |
| Cross section and side views of pile showing reinforcement and prestressing steel details and concrete dimensions |  |
| Notes including material specifications |  |

| III PS&E BRIDGE PLANS – 4) SUMMARY OF BORING LOGS | Checker Initials |
| --- | --- |
| Not required for WFLHD projects |  |
| Elevation view of each test hole |  |
| Plan view of structure locating and identifying each test hole |  |
| Typical test hole log symbology |  |
| Legend of test hole material types |  |
| Boring log notes |  |
| North arrow |  |
| III PS&E BRIDGE PLANS – 5) SLOPE PROTECTION | Checker Initials |
| #### A) RIPRAP |  |
| Existing contour plan |  |
| Plan view including substructure units, stream limits, and direction of flow |  |
| Horizontal limits |  |
| Cross section indicating slope, slope protection type, size, class, depth, geotextile type |  |
| Elevations at berm, toe, streambed, and other points as needed |  |
| North arrow |  |
| Existing bridge (if applicable) |  |
| #### B) CONCRETE SLOPE PAVING |  |
| Plan view showing horiz. limits dimensioned from reference lines |  |
| Cross section showing reinforcement details |  |
| Expansion and construction joint details |  |
| Label toe of slope, face of breastwall |  |
| III PS&E BRIDGE PLANS – 6) CONSTRUCTION SEQUENCING/STAGING | Checker Initials |
| Sequence of work with corresponding bridge sections illustrating the assumed routing of traffic and construction sequence |  |
| Identify existing structures as ghost lines and new construction with solid lines |  |
| Verify travel lane widths |  |
| Transverse widths of traveled ways, barriers, offsets, clearances |  |
| Temporary barrier and attachment details |  |
| Saw cut lines for existing structure and construction joints for new structure |  |
| Direction of traffic (or orientation of sections) |  |

| III PS&E BRIDGE PLANS – 7) ABUTMENTS | Checker Initials |
| --- | --- |
| 1. ABUTMENTS |  |
| General layout plan and elevation views |  |
| Pedestals, shear blocks, weepholes, utility blockouts, steel embedments |  |
| Bearings and joint fillers |  |
| Elevations at bearings, bottom of substructure units, top of walls, construction joints |  |
| Keys, fillets, chamfers |  |
| Typical sections |  |
| Drainage details (if applicable) |  |
| Identify berms and finished ground lines |  |
| North arrow |  |
| Piles/shafts size |  |
| Identify special seismic details (if applicable) |  |
| 2. WINGWALLS |  |
| Wingwall section |  |
| Wingwall elevation |  |
| Elevations at end and bottom of wingwalls |  |
| Weephole locations |  |
| 3. ENDWALLS |  |
| Plan View |  |
| Elevation view |  |
| Section view |  |
| Girder embedment |  |
| Shear block locations and details |  |
| Guard angle details |  |
| III PS&E BRIDGE PLANS – 9) PIERS/BENTS | Checker Initials |
| Pier layout |  |
| General layout plan and elevation views |  |
| Pedestals, shear blocks, blockouts, steel embedments |  |
| Bearings and joint fillers |  |
| Elevations at bearings, bottom of substructure units, construction joints |  |
| Keys, fillets, chamfers, column flairs |  |
| Identify finished ground lines |  |
| Typical sections |  |
| North arrow |  |
| Piles/shafts size |  |
| Identify special seismic details (if applicable) |  |
| III PS&E BRIDGE PLANS – 8) RETAINING WALLS | Checker Initials |
| #### A) CAST IN PLACE CONCRETE WALLS |  |
| General layout plan and elevation views, identifying construction and expansion joints, layout geometry |  |
| Length of wall sections and lateral offset values |  |
| Elevations and stations at top of wall at joint locations |  |
| Footing elevations |  |
| Typical sections |  |
| Counterfort details (if applicable) |  |
| Minimum bar embedment requirements |  |
| Wall batter, shear keys, sliding keys, blockout locations, weephole locations |  |
| Drainage details (if applicable) |  |
| Step details, expansion and contraction joint details (if applicable) |  |
| Wall notes |  |
| Construction sequence details (if applicable) |  |
| Stem Offset Detail |  |
| Excavation Limits Diagram (if applicable) |  |
| B) SHEET PILE WALLS AND TIEBACK WALLS |  |
| General layout plan and elevation views |  |
| Length of wall, vertical and horizontal spacing of anchors, spacing of expansion and contraction joints |  |
| Waler location and details |  |
| Elevations at bottom of sheet piles, top of cap/wall, existing ground line/streambed, and at water surface |  |
| Drainage details (if applicable) |  |
| Railing/parapet and connections (if applicable) |  |
| Existing structures and utilities (if applicable) |  |
| Wall cross sections |  |
| Anchor sizes, anchor slopes, lengths, plate sizes and connection details |  |
| Dewatering system requirements (if applicable) |  |

| III PS&E BRIDGE PLANS – 10) SUPERSTRUCTURE – A) PRESTRESSED CONCRETE GIRDERS AND BEAMS | Checker Initials |
| --- | --- |
| #### 1. GIRDER FRAMING PLAN |  |
| Geometry to locate girders |  |
| Label girders |  |
| Locate diaphragms |  |
| Dimension span lengths (parallel to girders) |  |
| 2. GIRDER ELEVATION |  |
| Length of girder |  |
| Transverse reinforcement including railing embedments |  |
| Locate CL bearing and CL lifting eye |  |
| Blockouts and connection inserts |  |
| Label strands |  |
| Locate strand hold-downs and debonding points (if applicable) |  |
| Endblocks (if applicable) |  |
| 3. GIRDER MISCELLANEOUS |  |
| Girder cross sections at midspan and end |  |
| Strand layout |  |
| Show embedments and verify clearances with strands and reinforcement |  |
| End zone details |  |
| Lifting eye detail (if applicable) |  |
| Provide Estimate Table with quantities for concrete, reinforcing steel, and strands (per girder for each unique girder) |  |
| Label beam (if applicable) |  |
| 4. DECK PLAN |  |
| Locate diaphragms |  |
| CL girders |  |
| Reinforcement cutoff locations for negative moment (if applicable) |  |
| Longitudinal and transverse bars, top and bottom, in deck slab including those that protrude into railing |  |
| Bar marks match those of Typical Section |  |
| Stage construction details (if applicable) |  |
| Superstructure Concrete Placement Diagram (if applicable) |  |
| Transverse Slab Construction Joint detail (if applicable) |  |
| III PS&E BRIDGE PLANS – 10) SUPERSTRUCTURE – A) PRESTRESSED CONCRETE GIRDERS AND BEAMS cont’d. | Checker Initials |
| 5. TYPICAL SECTION |  |
| Split section with pier section and midspan section |  |
| Show all girders, slab, railings, sidewalks, wearing surface, drains, utilities (including supports) |  |
| Dimension transversally out-to-out including widths of traveled ways, sidewalk, railings |  |
| Concrete diaphragm details including connection hardware |  |
| Longitudinal and transverse bars in slab including those that protrude into railing |  |
| Cross slopes including sidewalk |  |
| Slab thickness at midpsan, deck edge, and soffit |  |
| Label girders |  |
| Drip groove |  |
| Stage construction details including closure pours, keys, construction joints |  |
| III PS&E BRIDGE PLANS – 10) SUPERSTRUCTURE – B) STEEL GIRDERS | Checker Initials |
| #### 1. GIRDER FRAMING PLAN |  |
| Geometry to locate girders |  |
| Label girders |  |
| Locate diaphragms |  |
| Splice locations |  |
| Dimension span lengths (parallel to girders) |  |
| 2. GIRDER ELEVATION |  |
| Length of girder |  |
| Locate CL bearing, CL Piers, splice locations |  |
| Shear connectors and transverse stiffeners |  |
| Identify girder section or plate sizes |  |
| Weld details |  |
| Identify different steel grades (hybrid girders) |  |
| 3. GIRDER MISCELLANEOUS |  |
| Girder cross sections with plate sizes, weld details, copes |  |
| Shear connector detail |  |
| Bearing stiffener, transverse stiffener, longitudinal stiffener details |  |
| Field splice details (if applicable) |  |
| Diaphragm details including connection details |  |
| Dead Load Camber Diagram |  |
| #### 4. DECK PLAN |  |
| Locate diaphragms |  |
| CL girders |  |
| Reinforcement cutoff locations for negative moment (if applicable) |  |
| Longitudinal and transverse bars, top and bottom, in deck slab including those that protrude into railing |  |
| Bar marks match those of Typical Section |  |
| Stage construction details |  |
| Superstructure Concrete Placement Diagram (if applicable) |  |
| Transverse Slab Construction Joint detail (if applicable) |  |
| #### 5. TYPICAL SECTION |  |
| Split section with pier section and midspan section |  |
| Show all girders, slab, railings, sidewalks, wearing surface, diaphragms, drains, utilities (including supports) |  |
| Dimension transversally out-to-out including widths of traveled ways, sidewalk, railings |  |
| Longitudinal and transverse bars in slab including those that protrude into railing |  |
| Cross slopes including sidewalk |  |
| Slab thickness at midpsan, deck edge, and soffit |  |
| Label girders |  |
| Drip groove |  |
| Stage construction details including closure pours, keys, construction joints (if applicable) |  |
| III PS&E BRIDGE PLANS – 10) SUPERSTRUCTURE – C) CAST IN PLACE SLABS | Checker Initials |
| #### 1. DECK PLAN |  |
| Reinforcement cutoff locations for negative moment (if applicable) |  |
| Longitudinal and transverse bars, top and bottom in slab including those that protrude into railing |  |
| Bar marks match those of typical section |  |
| Stage construction details |  |
| Superstructure Concrete Placement Diagram (if applicable) |  |
| Transverse Slab Construction Joint detail (if applicable) |  |
| #### 2. TYPICAL SECTION |  |
| Split section with pier section and midspan section |  |
| Show slab, railings, sidewalks, wearing surface, utilities |  |
| Dimension transversally out-to-out including widths of traveled ways, sidewalk, railings |  |
| Longitudinal and transverse bars in slab including those that protrude into railing |  |
| Cross slopes including sidewalk |  |
| Slab thickness at midpsan, deck edge, and soffit |  |
| Drip groove |  |
| Stage construction details including closure pours, keys, construction joints |  |
| Void spacing (if applicable) |  |
| #### 3. MISCELLANEOUS |  |
| Dimension void layout (if applicable) |  |
| Cell drainage details (if applicable) |  |
| Dead Load Deflection Diagram |  |
| Hold down details for void form (if applicable) |  |
| III PS&E BRIDGE PLANS – 10) SUPERSTRUCTURE – D) CAST IN PLACE BOX GIRDERS | Checker Initials |
| #### 1. GIRDER FRAMING PLAN |  |
| Geometry to locate girders |  |
| Label girders |  |
| Locate diaphragms |  |
| Girder lengths (parallel to girder) |  |
| Cell drains |  |
| 2. GIRDER ELEVATION |  |
| Length of girder |  |
| Transverse and longitudinal reinforcement |  |
| Blockouts and inserts |  |
| Post-tensioning profile (locate low point, high point, and inflection points as applicable) |  |
| #### 3. GIRDER MISCELLANEOUS |  |
| Construction Joint details |  |
| Abutment diaphragm details including post-tensioning anchorage reinforcement in end block |  |
| Pier diaphragms |  |
| Anchorage blockouts |  |
| Verify clearances between reinforcement and post-tensioning ducts |  |
| Notes including Concrete, Prestressing, Design Assumptions, Construction, Submittal Requirements |  |
| Access door reinforcement details (if applicable) |  |
| Access door details (if applicable) |  |
| Verify opening size of access openings (if applicable) |  |
| Locate access door for optimum inspection accessibility (if applicable) |  |
| Cell drain details |  |
| Dead Load Camber Diagram |  |
| #### 4. DECK PLAN |  |
| Locate diaphragms |  |
| Identify CL girders |  |
| Longitudinal and transverse (top and bottom) bars in deck slab including those that protrude into railing |  |
| Bar marks match those of typical section |  |
| Stage construction details |  |
| Construction joint details (if applicable) |  |

| III PS&E BRIDGE PLANS – 10) SUPERSTRUCTURE – D) CAST IN PLACE BOX GIRDERS cont’d. | Checker Initials |
| --- | --- |
| #### 5. TYPICAL SECTION |  |
| Split section with pier section and midspan section |  |
| Show all girders, slab, railings, sidewalks, wearing surface, drains, utilities (including supports) |  |
| Dimension transversally out-to-out including widths of traveled ways, sidewalk, railings |  |
| Longitudinal and transverse bars in slab including those that protrude into railing |  |
| Cross slopes including sidewalk |  |
| Top and bottom slab thickness, web thickness |  |
| Web slopes and fillets |  |
| Construction joints |  |
| Label girders |  |
| Drip groove |  |
| Scupper/drain details (if applicable) |  |
| Identify different concrete classes (if applicable) |  |
| III PS&E BRIDGE PLANS – 10) SUPERSTRUCTURE – BEARINGS & EXPANSION JOINTS | Checker Initials |
| #### E) BEARINGS |  |
| Identify bearings (fixed or expansion) |  |
| Quantity of each bearing |  |
| Grout pads (if applicable) |  |
| Detail bearing pads including layer thickness, overall thickness, shims required, hardness, width and length and orientation to CL bearing and CL girder |  |
| Geometry to locate bearings including orientation to structure |  |
| Sole plates, and bearing plates (if applicable) |  |
| Bearing notes to include bearing pad specs (required shear modulus and design method) |  |
| #### F) EXPANSION JOINTS |  |
| Geometry to locate joint |  |
| Table with temperature settings and joint design movements |  |
| Steel extrusion and anchorage details |  |
| Section, Plan, and Elevation |  |
| Seal type |  |

| III PS&E BRIDGE PLANS – 11) AESTHETICS | Checker Initials |
| --- | --- |
| #### A) STONE MASONRY |  |
| Locate and identify working lines |  |
| Critical stone dimensions including bulge and size requirements |  |
| Mortar requirements including joint widths, bed depth, and stone embedment |  |
| Hardware details including dovetail slots, anchor types and configurations, anchor bolt or rebar sizes |  |
| Cross sections and details where attachment details are unique |  |
| Dimension Stone Layout (if applicable) |  |
| Notes including material requirements, stone specifications, construction requirements, etc. |  |
| Dovetail slot spacing on the appropriate drawings |  |
| #### B) SURFACE TREATMENTS |  |
| Locate and orient lettering to reference lines (if applicable) |  |
| Horizontal and vertical size of letters, and recess depth |  |
| Formliner working lines and critical dimensions |  |
| Identify all surfaces that are to receive surface treatments |  |
| Identify all components to receive integral coloring or painting |  |
| III PS&E BRIDGE PLANS – 12) SIDEWALKS AND RAILINGS | Checker Initials |
| #### A) SIDEWALKS |  |
| General layout plan view |  |
| Typical section |  |
| Longitudinal and transverse bars including those protruding into railing |  |
| Score joint and contraction joint details |  |
| Chamfers |  |
| #### B) BRIDGE RAILINGS |  |
| Elevation view from end of wing to end of wing, dimensioning (along gutterline) post spacing, joint locations, splice locations, panel lengths |  |
| Expansion and contraction joint details (if applicable) |  |
| Typical sections at post and at unique sections |  |
| Verify crash tested requirements (if applicable) |  |
| Chamfers |  |
| Embedded plates, anchorages, blockouts, utility conduits, formed holes for transition railing connections |  |
| Post connection details |  |
| Detail welds, bolted connections, splices, and fabrication details |  |
| Concrete endblock (if applicable) |  |
| Isometric view of endblock (if applicable) |  |
| Verify pedestrian screen (if applicable) |  |
| Railing notes (if applicable) |  |
| C) TRANSITION RAILINGS |  |
| Elevation and plan views dimensioning length, post spacing, offsets, pay length |  |
| Connection between endblock or bridge railing and transition railing |  |
| Terminal connector, connection hardware, railing elements, post and blockout sizes |  |
| Transition notes (if applicable) |  |
| Verify crash tested requirements (if applicable) |  |
| III PS&E BRIDGE PLANS – 13) APPROACH SLABS | Checker Initials |
| Skew |  |
| Longitudinal and transverse bars, top and bottom, in Plan View |  |
| Reinforcement, bar clearances, and slab depth in Section View |  |
| Identify joint filler locations |  |
| Detail end of approach slab for movement if required (Type II) |  |
| Sleeper slab details (if applicable) |  |
| III PS&E BRIDGE PLANS – 14) BAR LIST | Checker Initials |
| Heading for each structure element |  |
| Bar mark, bar size, bar bend type, pin diameter, location of bar (this portion only for contract plans for WFLHD) |  |
| Number of each bar, length, weight, dimensions, bend diagram, weight subtotal for each heading (this portion along with above information provided to construction for WFLHD) |  |
| Identify epoxy coated bars |  |
| Bar list notes |  |
| Added note for bar elements that are included in separate pay items |  |