US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 201 Clearing and Grubbing

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | General |
|  | 1. Is erosion control constructed? (201.03) |
|  | 2. Was work performed within designated limits? (201.03) |
|  | 3. Has vegetation designated to remain been damaged? (201.03) |
|  | 4. Has vegetation adjacent to water been damaged? (201.03) |
|  | 5. Has cut or damaged trees and shrubs been treated with wound dressing? (201.03) |
|  | Clearing |
|  | 6. Where trees cut so they fall within clearing limits? (201.04a) |
|  | 7. Are stumps cut flush with or below final groundline in cut slope rounding areas?(201.04b) |
|  | 8. Are stumps cut within 6 inches of ground in areas outside excavation, embankment and slope rounding limits? (201.04c) |
|  | 9. Are branches that extend over the road surface and shoulders trimmed to attain height of 20 feet? (201.04d) |
|  | 10. Are branches that have been cut treated with wound dressing? (201.04d) |
|  | Grubbing |
|  | 11. Are all areas grubbed in excavation area, except for cut slope rounding areas? (201.05a) |
|  | 12. Are all embankment areas grubbed? (201.05b) 12B. Were undisturbed stumps cut to protrude no more then 6 inches above original ground and covered with more then 4 feet of embankment? (201.05b) |
|  | 13. Are pits, channel changes and ditches grubbed to depth necessary for excavation? (201.05c) |

|  | 14. Are stump holes and other grubbing holes filled to the level of the surrounding ground per subsection 209.10 and compacted per subsection 209.11? (201.05d) |  |
| --- | --- | --- |
|  | Disposal |  |
|  | 15. Is clearing and grubbing debris disposed of according to Subsection 203.05? (201.06) |  |
|  | 17. |  |
|  | 18. |  |
|  | 19. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: