US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 204 Excavation and Embankment

| Project Name: |  |  | Project No.: |
| --- | --- | --- | --- |
| Date: | Weather: |  |  |
| Contractor: |  | Subcontractor: |  |
| Inspector: |  | Location/Station: |  |
| Description of work being inspected: |  |  |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Preparation for Roadway Excavation and Embankment |
|  | 1. Is the area clear of vegetation and obstructions? (204.04) |
|  | Conserved Topsoil |
|  | 2. Has topsoil been conserved, stockpiled and separated properly in the approved location? (204.05) |
|  | Roadway Excavation |
|  | 3. Has material and vegetation been disturbed outside the construction limits? (204.06a) |
|  | 4. Are embankments built from suitable material? (204.06a) |
|  | 5. At the end of each day’s operation, was the area prepped correctly for water runoff? (204.06a) |
|  | 6. On rock cuts, was blasting performed per section 205 and fill compacted per subsection 204.11? (204.06b) |
|  | 7. On earth cuts was earth scarified to 6 inches below subgrade and then compacted per subsection 204.11? (204.06c) |
|  | Subexcavation |
|  | 8. Is material excavated to limits designated by CO? (204.07) |
|  | 9. Have cross sections been taken according to subsection 152.03b? (204.07) |
|  | 10. Has unsuitable material been kept separately and not mixed with backfill? (204.07) |
|  | 11. Has unsuitable material been disposed of per subsection 204.14? (204.07) |
|  | 12. Was subexcavation backfilled with topping or other suitable material? (204.07) |
|  | 13. Is backfill compacted per subsection 204.11? (204.07) |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | 14. Was all suitable roadway excavation used in embankment? (204.08) |  |
|  | 15. Was borrow from an acceptable source used per subsection 105.02? (204.08) |  |
|  | Preparing Foundation for Embankment Construction |  |
|  | 16. When embankment was less than 4 feet high was topsoil removed, ground scarified to a depth of 6 inches and then compacted per subsection 204.11? (204.09a) |  |
|  | 17. When embankment was over existing asphalt, concrete or gravel road surface was gravel scarified to 6 inches, asphalt and concrete scarified or pulverized to 6 inches below pavement reducing all particles to a maximum size of 6 inches, and then compacted per subsection 204.11? (204.09b) |  |
|  | 18. When embankment is across ground not capable of supporting equipment were successive loads of material distributed uniformly in layer thickness to minimum depth necessary to support equipment? (204.09c) |  |
|  | 19. When embankment on an existing slope steeper than 1V:3H were horizontal benches cut in the existing slope so each bench began at the intersection of the original ground and the vertical cut of the previous bench? (204.09d) |  |
|  | Embankment Construction |  |
|  | 20. Were embankment side slopes compacted with a tamping foot roller, by walking with a dozer or by over-building and removing excess material to the final slope? (204.10a) |  |
|  | 21. When embankment was placed on one side of abutments, wing walls, piers or culvert headwalls was it placed correctly to prevent excessive pressure against the structure? (204.10a) |  |
|  | 22. Were structural piles where placed in embankment location was the particle size kept to a maximum of 4 inches? (204.10a) |  |
|  | 23. Within the road prism was embankment material placed in horizontal layers not exceeding 12 inches in compacted thickness? (204.10b) |  |
|  | 24. Within the road prism if boulders and rock fragments to large for 12-inch layers were used was sufficient earth and smaller rocks used to fill voids? (204.10b) |  |
|  | 25. If rock fragments and boulders greater than 24 inches in diameter were used were the following criteria met: (1) reduce rock to less than 48 inches in the largest dimension, (2) Distribute rock within the embankment to prevent nesting, (3) embankment material placed around each rock filling the voids? (204.10c) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 26. Were placing embankment outside of roadway prism was material place in horizontal layers not exceeding 24 inches in compaction thickness? (204.10d) |  |
|  | Sloping, Shaping and Finishing |  |
|  | 27. When viewed from the road does slope have a uniform roughened surface? (204.13a) |  |
|  | 28. If slope is solid rock have the tops and bottoms been rounded? This includes the slopes of drainage ditches. (204.13a) |  |
|  | 29. On stepped slopes (when required) are the steps constructed 1 1/3V:1H to 1V:2H and approximately 18 inches high? Are they also blended into the natural ground? (204.13b) |  |
|  | 30. Is subgrade shaped to a smooth surface and to the cross-section, while also showing a gradual transition into slope adjustments without noticeable breaks? (204.13c) |  |
|  | 31. When finishing, was material larger than 6 inches and located 6 inches from the top of the roadbed removed? (204.13d) |  |
|  | 32. Does the finish meet the following criteria: (1) roadbed compacted according to subsection 204.11(b) and (c), or roadbed compacted according to subsection 204.11(a) as per plans. (204.13d) |  |
|  | 33. Are ditches finished to within +or- 0.10 feet of the staked line and grade? (204.13d) |  |
|  | 34. |  |
|  | 35. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: