‘US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 207 Earthwork Geotextiles

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Geotextiles General |
|  | 1. Were trees and shrubs cut flush with ground before placing Geotextiles on native ground? (207.03) |
|  | 2. Was topsoil and vegetation mat left in place? (207.03) |
|  | 3. Were sharp objects and large rocks removed before placing Geotextiles? (207.03) |
|  | 4. Were depressions and holes filled with suitable material before placing Geotextiles? (207.03) |
|  | 5. Were damaged areas of Geotextile repaired with Geotextile patch overlapping 3 feet beyond damaged area? (207.03) |
|  | Separation and Stabilization Applications |
|  | 6. Where Geotextile is used on subgrade, was subgrade prepared according to subsections 204.13c and d? (207.04) |
|  | 7. Was the Geotextile placed smooth and free of tension, stress and wrinkles? (207.04) |
|  | 8. Was Geotextile folded or cut to conform to curves? (207.04) |
|  | 9. Was Geotextile overlapped a minimum of 2 feet at all joints and overlapped in the direction of construction? (207.04) |
|  | 10. Were Geotextile overlaps not placed in anticipated longitudinal wheel load areas? (207.04) |
|  | 11. Was Geotextile anchored with pins, staples or piles of cover material? (207.04) |
|  | 12. Was cover material placed without running equipment on Geotextile mat? (207.04) |
|  | 13. Was cover material placed with a minimum lift thickness of 12 inches and compacted with a rubber tired or non-vibratory smooth drum roller? (207.04) |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | 14. If ruts caused by construction equipment were present, were they repaired with cover material and not just regraded with placement equipment? (207.04) |  |
|  | 15. If foundation failures occurred in subsequent lifts were repairs performed and compacted with a non-vibratory compactor? (207.04) |  |
|  | Permanent Erosion Control Applications |  |
|  | 16. For slope or wave protection was Geotextile placed so the long dimension runs downhill? (207.05) |  |
|  | 17. For stream protection was Geotextile placed so the long dimension runs parallel to centerline of channel? (207.05) |  |
|  | 18. Where Geotextile needed to be overlapped was the overlap distance a minimum of 12 inches and the uphill or upstream sheet over the downhill or down stream sheet? (207.05) |  |
|  | 19. Was aggregate, slope protection or riprap placed over Geotextile starting at the toe of the slope? (207.05) |  |
|  | 20. When riprap was placed over Geotextile was it dropped from a height of more than 12 inches? (207.05) |  |
|  | 21. When slope protection rock or aggregate was placed over Geotextile was it dropped from a height of more than 3 feet? (207.05) |  |
|  | 22. When Geotextile was placed underwater was the cover material placed the same day? (207.05) |  |
|  | 23. |  |
|  | 24. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: