US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 208 Structure Excavation and Backfill for Selected Major Structures

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Preparation for Structure Excavation |  |
|  | 1. Was area cleared of vegetation and obstructions according to sections 201 and 203? (208.03) |  |
|  | General |  |
|  | 2. Were trenches or foundation pits excavated to a width and length that allowed room to work? (208.04) |  |
|  | 3. When excavation was completed, was approval given as to the character and suitability of the foundation material? (208.04) |  |
|  | 4. If blasting was necessary, was it performed per Section 205? (208.04) |  |
|  | 5. Were OSHA safety regulation (29 CFR, part 1926, subpart P, Excavation) followed for safety when sloping the sides of excavations, using shoring and bracing and other safety features? (208.04) |  |
|  | 6. If sides of excavation needed to be sloped for safety, was a copy of the design submitted? (208.04) |  |
|  | 7. If support systems, shield systems or other protective systems were used, was the shoring designed per section 562 and were working drawings and construction details per section 104.03 submitted? (208.04) |  |
|  | 8. When safety features were no longer needed were shoring and bracing removed at least 2 feet below finished ground? (208.04) |  |
|  | 9. If existing pavement and concrete structures were adjacent to the area were they saw cut? (208.04) |  |
|  | 10. Was suitable material from excavation conserved to be used as backfill? (208.04) |  |
|  | 11. If a waterway was close by, was excavated material stored in or near water? (208.04) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 12. Was stockpiled excavated material and equipment kept at least 2 feet from excavation? (208.04) |  |
|  | 13. Was unsuitable material disposed of per section 204.14? (208.04) |  |
|  | 14. Was excess water removed from excavation before work was performed? (208.04) |  |
|  | Channel Preservation |  |
|  | 14A-1. When near or in running water was excavation done inside of cofferdams, sheeting, or other separation such as dikes or sandbags? (208.05(a)) |  |
|  | 14A-2. Was the natural bed of the waterway adjacent to the work left undisturbed? (208.05(b)) |  |
|  | 14A-3. Was the excavation backfilled with structural backfill to original groundline? (208.05(c)) |  |
|  | Cofferdams |  |
|  | 14B-1. Was a cofferdam used when excavation was under water or affected by ground water? (208.06) |  |
|  | 14B-2. Where working drawing submitted showing proposed methods and construction details of the cofferdam according to Subsection 104.03 and Section 562? (208.06) |  |
|  | 14B-3. Did the working drawings show and does the cofferdam conform to paragraphs (a) thru (g) of Subsection 208.06. (208.06) |  |
|  | 14B-4. Was the cofferdam and all material removed down to natural bed of the waterway when no longer required? (208.06) |  |
|  | 14B-5. Was cofferdam material outside the waterway removed to minimum of 2 feet below the surface of the finished ground? (208.06) |  |
|  | Foundation Seal |  |
|  | 14C-1. If a seal is required was a design in accordance with Subsection 552.03 for seal concrete completed? (208.07) |  |
|  | 14C-2. Was seal concrete placed in accordance with Section 552? (208.07) |  |
|  | 14C-3. Was the water level inside the cofferdam maintained at the same level at the outside water level while placing the foundation seal? (208.07) |  |
|  | 14C-4. If placing seal in tidal water was a vent or port placed in the cofferdam at low water level? (208.07) |  |
|  | 14C-5. Was the concrete sealed cofferdam dewatered after the concrete strength was sufficient to withstand the hydrostatic pressure? (208.07) |  |
|  | Dewatering |  |
|  | 14D-1. While placing concrete were the pumps located and operated outside the foundation forms? (208.08) |  |
|  | 14D-2. While pumping from the interior of the cofferdam was it performed in a manner to avoid the disturbance of the concrete material? (208.08) |  |
|  | Foundation Preparation |  |
|  | 15. If footings placed on bedrock, was the bottom cut to the specific elevation: level, stepped or serrated? (208.09a) |  |
|  | 16. If footings placed on bedrock, were the foundation surfaces cleaned and all seams and crevices grouted? (208.09a) |  |
| Yes No N.A. | Foundation Preparation (Continued) |  |
|  | 17. If footings placed on an excavated surface other than bedrock, was the bottom undisturbed? (208.09b) |  |
|  | 18. If footings placed on an excavated surface other than bedrock, was material removed to grade and compacted immediately prior to concrete placement? (208.09b) |  |
|  | 19. If footings keyed into undisturbed material, was the foundation excavated to the limits of the footing and compacted? (208.09c) |  |
|  | 20. If footings keyed into undisturbed material and material did not stand vertical, were all spaces between the limits of the footing and the remaining undisturbed material filled with concrete? (208.09c) |  |
|  | 21. If unstable material was encounter below footing elevation, was unstable material removed and then replaced with foundation fill in 6 inch compacted layers and compacted according to subsection 208.11? (208.09d) |  |
|  | 22. If foundation used piles, was foundation first excavated to elevation and then the piles driven? (208.09e) |  |
|  | 23. After piles were installed was the bed to receive the footings smoothed and compacted? (208.09e) |  |
|  | Backfill |  |
|  | 24. Was backfill placed in horizontal layers that did not exceed 6 inches in compacted thickness and compacted per subsection 208.11? (208.10) |  |
|  | 25. Was structural backfill brought up evenly on all sides of structure and extended to the limits of the excavation or to natural ground? (208.10) |  |
| Conformance | CHECKS (characteristics) |  |
|  | 26. Was concrete allowed to cure for 7 days or reach 80 percent of design strength before backfilling? (208.10) |  |
|  | 27. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: