US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 209 Structure Excavation and Backfill

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Preparation for Structure Excavation |  |
|  | 1. Was area cleared of vegetation and obstructions according to sections 201 and 203? (209.03) |  |
|  | General |  |
|  | 2. Were trenches or foundation pits excavated according to subsection 208.04? (209.04) |  |
|  | 3. Was foundation grade excavated without unduly disturbing the trench or foundation surface? (209.04) |  |
|  | 4. Was foundation compacted? (209.04) |  |
|  | Foundation Preparation |  |
|  | 5. If any unsuitable material was found at foundation grade was it replaced with foundation fill? (209.08) |  |
|  | 6. Was foundation fill material placed and compacted according to subsection 208.09d? (209.08) |  |
|  | Bedding |  |
|  | 7. For box culverts and structures other than pipe culverts, when required was bedding material placed and shaped in compacted layers not exceeding 6 inches in depth and compacted according to section 209.11? (209.09a) |  |
|  | 8. For pipe culverts, was the foundation level? (209.09b) |  |
|  | 9. For pipe culverts, was un-compacted bedding material placed in a layer of uniform thickness, 4 inches for 12 to 54 inch diameter and 6 inches for larger than 54 inch diameter culverts, and the culvert placed on the un-compacted bedding? (209.09b) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Backfill |  |
|  | 10. Was backfill placed in layers when compacted did not exceed 6 inches? (209.10a) |  |
|  | 11. Was each layer compacted according to subsection 209.11? (209.10a) |  |
|  | 12. Was backfill built up evenly on all sides and extended to the limits of excavation or natural ground? (209.10a) |  |
|  | 13. Was concrete allowed to cure for 7 days or until 80 percent of design strength was achieved? (209.10a) |  |
|  | 14. If pipe culvert with compacted backfill, was material placed and compacted to a height of 12 inches above the top of the pipe? (209.10b) |  |
|  | 15. If pipe culvert with compacted backfill, when backfill reached 12 inches above the top of the pipe, was the remainder of trench filled with suitable roadway excavation or unclassified borrow, in compacted layers not more than 6 inches, according to subsection 209.11? (209.10b) |  |
|  | 15B. If pipe culvert with lean concrete backfill, was the pipe staked to prevent floating and movement? (209.10(b)(2)) |  |
|  | 15C. If pipe culvert with lean concrete backfill, was pipe backfilled using lean concrete according to Section 614? (209.10(b)(2)) |  |
|  | 16. If backfilling arch culverts with headwalls before headwalls are in place, was backfill placed and compacted according to subsection 209.10c (1)? (209.10c) |  |
|  | 17. If backfilling arch culverts with headwalls after headwalls are in place, was backfill placed and compacted according to subsection 209.10c (2)? (209.10c) |  |
|  | 18. When patching existing pavement areas was the patch constructed where the top 15 inches was 12 inches of crushed aggregate according to section 301 and 3 inches of asphalt concrete according to section 404 or 417? (209.10d) |  |
|  | 19. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: