US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 212 Linear Grading

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Roadway Preparation |
|  | 1. Was area cleared of vegetation and obstructions according to sections 201 and 203? (212.02) |
|  | Roadway Excavation and Embankment |
|  | 2A. Was the roadbed constructed in accordance with Section 204 as modified by Section 212? (212.03) |
|  | 2. Was embankment material at a moisture content suitable for compaction? (212.02) |
|  | 3. Was embankment material placed in 12-inch layers and compacted according to subsection? (212.02) |
|  | 4. Were approaches to all existing roads, parking areas and trails constructed? (212.02) |
|  | Grading Tolerance |
|  | 5. Was alignment (centerline) shifted and if so, was the shift kept under 10 feet left or right of planned centerline? (212.04) |
|  | 6. Was the curve radii reduced and if so, was it by no more than 50 percent and is the radius greater than 100 feet? (212.04) |
|  | 7. Was profile grade shifted and if so did it exceed a maximum of 5 feet up or down from plan or did grade tangent vary more than 2 percent from plan? (212.04) |
|  | 8. If profile was adjusted were forward and back grade tangents connected with a uniform vertical curve consistent with the design? (212.04) |
|  | 9. Were all alignment changes made without encroaching or impacting stream channels, wetlands, or extending beyond right-of-way or easement limits? (212.04) |

| Yes No N.A. |  |  |
| --- | --- | --- |
|  | 14. 10. Was drainage not adversely affected by alignment or profile changes? (212.04) |  |
|  | 15. |  |
|  | 16. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: