US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 251 Riprap

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. Was slope dressed to form a smooth surface? (251.03) |  |
|  | 2. If Geotextile required was it placed according to section 207? (251.03) |  |
|  | Place Riprap |  |
|  | 3. Was Riprap placed to full thickness in one operation to avoid displacing underlying material and avoid causing segregation or damage to the prepared surface? (251.04) |  |
|  | 4. Were rocks place or rearrange to obtain a dense uniform blanket with a reasonably smooth surface? (251.04) |  |
|  | Keyed Riprap |  |
|  | 5. Was Riprap set into place by exerting impact pressure, while not reducing the effective size of the rock? (251.05) |  |
|  | 6. Was impact pressure used on Riprap under water? (251.05) |  |
|  | Grouted Riprap |  |
|  | 7. Was rock placed according to Subsections 251.04 or 251.05? (251.06) |  |
|  | 8. Were rocks washed and thoroughly moistened before placing grout? (251.06) |  |
|  | 9. Was grout placed at air temperatures of 35º or above in the near surface voids? (251.06) |  |
|  | 10. Was the grout placed in a way to prevent segregation? (251.06) |  |
|  | 11. Was the grout started at the lowest elevation of the grout? (251.06) |  |
|  | 12. Did the grout fill all voids without unseating the rocks? (251.06) |  |
|  | 13. Was grout allowed to cure for 3 days before placing additional rock or grout layers? (251.06) |  |
|  | 14. Were wipe holes provided if required? (251.06) |  |
| Yes No N.A. | CHECKS (characteristics) |  |
|  | 15. Was the grouted riprap kept moist for 3 days and kept from freezing for 7 days after grouting was completed? (251.06) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: