US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 253 Gabions and Revet Mattresses

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. Were the areas surveyed according to section 152 and were the limits of the wall installation verified? (253.03) |  |
|  | 2. Were installation drawings submitted per subsection 104.03? (253.03) |  |
|  | Basket Assembly |  |
|  | 3. Was basket wire coating damaged during construction? (253.04) |  |
|  | 4. Were vertical edges fastened according to subsection 253.05? (253.04) |  |
|  | 5. Were the diaphragms rotated into position and joined to the vertical edges according to subsection 253.05? (253.04) |  |
|  | Structure Erection |  |
|  | 6. While placing the empty gabion baskets on the foundation, were the adjacent baskets interconnected along the top and vertical edges using permanent fasteners? (253.05) |  |
|  | 7. When using the following methods to fasten baskets together were they installed according to subsection 253.05: Lacing wire, spiral binders or alternate fasteners? (253.05) |  |
|  | 8. Were the vertical layers of baskets interconnected to the underlying layers in the same manner along the front, back and sides? (253.05) |  |
|  | 9. Were joints between baskets of adjacent rows and layers staggered by at least on cell length? (253.05) |  |
|  | Cell Filling |  |
|  | 10. Were baskets properly aligned and all kinks and folds removed from wire mesh? (253.06) |  |
|  | 11. Was rock carefully placed in the basket cells to prevent bulging of the baskets and to minimize voids in rock fill while maintaining basket alignment? (253.06) |  |
|  | 12. Were internal connecting wires used in each unrestrained exterior basket cell that was greater than 1 foot in height, including basket cells left temporarily unrestrained? (253.06) |  |
|  | 13. Were internal connecting wires placed concurrently with rock placement? (253.06) |  |
|  | 14. Were wire cells filled so that any row or layer was not filled more than 1 foot above any adjacent cell and this process repeated until the baskets were full and the lid laid on the final rock layer? (253.06) |  |
|  | 15. Were lids secured to the sides, end, and diaphragms according to section 253.05? (253.06) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 16. Were all exposed basket surfaces smooth and neat with no sharp rock edges projecting through the wire mesh? (253.06) |  |
|  | Backfilling |  |
|  | 17. Was Geotextile placed over the back face of the gabion structure and backfilled with structural backfill concurrently with basket fill according to subsection 209.10? (253.07) |  |
|  | 18. Was each layer compacted according to subsection 209.11 except within 3 feet of the gabion structure where an acceptable lightweight mechanical or vibratory compactor should have been used? (253.07) |  |
|  | Rivet Mattresses |  |
|  | 19. Were mattresses placed in accordance with Subsection 253.08? (251.08) |  |
|  | 20. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: