US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 255 Mechanically Stabilized Earth Walls

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. Were the areas surveyed according to section 152 and were the limits of the wall installation verified? (255.03) |  |
|  | 2. Were installation drawings submitted per subsection 104.03? (255.03) |  |
|  | 3. Was the foundation graded for a width equal to the length of reinforcing elements plus 18 inches? (255.03) |  |
|  | 4. If the wall foundation was set on rock was 6 inches of select granular backfill placed under the reinforcing mesh or strips? (255.03) |  |
|  | 5. For concrete-faced walls, was a precast reinforced or a non-reinforced cast-in-place concrete leveling pad provided? (255.03) |  |
|  | 6. If cast-in-place was used, was it allowed to cure for a minimum of 12 hours before placing wall panels? (255.03) |  |
|  | Wall Erection |  |
|  | 7. Were walls erected according to the drawings and manufactures recommendations? (255.04) |  |
|  | 8. If a manufacturing representative was requested to be on site, was one available? (255.04) |  |
|  | 9. If concrete faced wall, were panels erected by means of a lifting device connected to the upper edge of the panel? (255.04) |  |
|  | - If concrete-faced walls, were the following tolerances met: Panels aligned within ¾ inch vertically and horizontally when measured with a 10 foot straight edge, Joint openings ¾ +or- ¼ inch wide and Overall vertical tolerance (top to bottom) of ½ inch maximum over 10 feet of wall height? (255.04a) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 11. If concrete-faced walls, were all joints covered on the backside of the panels with a 12-inch wide strip of Geotextile and if Geotextile needed overlapped splices, was it overlapped a minimum of 4 inches? (255.04a) |  |
|  | 12. If concrete-faced wall, were panels held in position with temporary wedges or bracing during backfilling operations? (255.04a) |  |
|  | 13. If wire-faced wall, were backing mats and1/4 inch hardware cloth placed in successive horizontal lifts as backfill placement proceeded? (255.04b) |  |
|  | 14. If wire-faced wall, were the soil reinforcement elements connected, tightened and anchored to the wall facing units before placing backfill? (255.04b) |  |
|  | 15. If wire-faced wall, was an individual lift tolerance and an overall wall (top to bottom) vertical tolerance of 1 inch per 10 feet of wall height exceeded? (255.04b) |  |
|  | 16. If wire-faced wall, did reinforcement elements get placed within 1 inch vertically above the corresponding connection elevation at the wall face? (255.04b) |  |
|  | 17. If wire-faced wall, were reinforcing elements placed below the corresponding connection elevations? (255.04b) |  |
|  | 18. If wire-faced wall, were the following deviations exceeded: (1) the designed batter of the wall by more than 1 inch per 10 feet of wall height, (2) More than 2 inches at any point in the wall from a 10 foot straight edge placed on the theoretical plane of the design face? (255.04b) |  |
|  | 19. If Gabion-faced wall, was the first lift of backfill placed before filling the Gabion basket? (255.04c) |  |
|  | 20. If Gabion-faced wall, was wall constructed according to section 253? (255.04c) |  |
|  | 21. If Gabion-faced wall, was reinforcement mesh laid horizontally on compacted fill and normal to the face of the wall? (255.04c) |  |
|  | 22. If Gabion-faced wall, was the Gabion facing unit connected to reinforcement mesh with spiral binders or tie wire at 4 inch nominal spacing with alternating single and double locked loops? (255.04c) |  |
|  | 23. If Gabion-faced wall, was the reinforcement mesh pulled and anchored taut before placing additional backfill? (255.04c) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Backfilling |  |
|  | 24. Was backfilling of the stabilized volume performed according to subsection 209.10? (255.05) |  |
|  | 25. Were all voids filled below the reinforcing mesh or strips? (255.05) |  |
|  | 26. Was each layer compacted according to subsection 209.11, except when within 3 feet of the wall face and then was an acceptable lightweight mechanical or vibratory compactor used? (255.05) |  |
|  | 27. Where the stabilized volume supports spread footings for bridges or other structural loads was the top 5 feet compacted to at least 100 percent of the maximum density? (255.05) |  |
|  | 28. Were the facing or reinforcing elements damaged or disturbed? (255.05) |  |
|  | 29. Was equipment allowed to operate directly on top of the reinforcing mesh or strips? (255.05) |  |
|  | 30. Were all damaged, misaligned or distorted wall elements corrected? (255.05) |  |
|  | 31. Was backfilling and compaction behind the stabilized volume performed according to subsection 209.10? (255.05) |  |
|  | 32. At the end of the day’s operation, was the last lift of backfill sloped away from the wall face? (255.05) |  |
|  | 33. Was surface runoff from adjacent areas allowed to enter the wall construction area? (255.05) |  |
|  | 34. |  |
|  | 35. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: