US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 260 Rock Bolts

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Submittal |  |
|  | 1A-1. Was the information required by Subsection 260.03 submitted at least 14 days before installation of the rock bolts? (260.03) |  |
|  | 1A-2. Were the submittals approved before work on the rock bolts started? (260.03 |  |
|  | Rock Bolt |  |
|  | 1B. Was the bolt sized so the design load did not exceed 60 percent of the minimum guaranteed ultimate strength of the bolt? (260.04) |  |
|  | 2. Were the bolts selected and fabricated to carry the specified load? (260.04) |  |
|  | 3. If bolts needed to be coupled were they coupled according to subsection 260.04? (260.04) |  |
|  | Handling and Storage |  |
|  | 4. Were bolts protected from dirt, mud, water and other harmful substances to avoid damage and corrosion? (260.05) |  |
|  | 5. Were rock bolts that were heavily corroded, pitted, damaged due to welding or showing signs of abrasions, cuts or nicks used? (260.05) |  |
|  | Installation |  |
|  | 6. Were rock bolts installed according to subsection 260.06? (260.06) |  |
|  | Tensioning |  |
|  | 7. Was tension applied to the bolt with a calibrated torque wrench to 125 percent of the design load, and after tensioning did the bolt get locked off at the specified design load? (260.07) |  |
|  | 8. If a polyester resin-anchored bolt was used did it get tensioned immediately after the fast-set resin in the bond zone set up and before the slow-set resin in the unbonded zone set up? (260.07) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 9A. If a grouted anchored bolt was used, did it get tensioned after the bonded length has been grouted and the grout has set up but before the unbonded length is grouted? (260.07) |  |
|  | 9B. If a mechanically anchored rock bolts was used did it get tensioned immediately after insertion into the drill hole according to manufacturer’s recommendations? (260.07) |  |
|  | 10. Was the bolt allowed to remain in place if: (a) The bolt was tensioned to 125 percent of the design load and the load held for 10 minutes, (b) The design load was maintained after final grouting was accomplished? (260.07) |  |
|  | 11. Were bolts replaced that did not meet (a) or (b) in a manner and in a location approved by the CO? (260.07) |  |
|  | 12. |  |
|  | 13. |  |
|  | 14. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: