US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 301 Untreated Aggregate Courses

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. Was the surface on which the aggregate course was placed, prepared according to section 204 or 303 as applicable? (301.03) |  |
|  | 2. After a representative quantity of aggregate was produced, were proposed target values submitted for the appropriate sieve sizes along with a representative 300-pound sample at least 14 days before incorporating the aggregate into the work? (301.03) |  |
|  | 3. Were target values set within the gradation ranges shown in table 703-2 or 703-3 for the required grading? (301.03) |  |
|  | Mixing and Spreading |  |
|  | 4. Was the optimum moisture content determined according to AASHTO T 180, Method D? (301.04) |  |
|  | 5. Was the aggregate mixed and moisture adjusted to obtain a uniform mixture within 2 percent of the optimum moisture content? (301.04) |  |
|  | 6. Was the mixture spread and shaped on the prepared surface in a uniform layer that did not exceed 6 inches of compacted thickness? (301.04) |  |
|  | 7. When more than one layer was necessary, was each layer compacted according to subsection 301.05 before placing the next layer? (301.04) |  |
|  | 8. Was hauling equipment routed uniformly over the full width of the surface to minimize rutting or uneven compaction? (301.04) |  |
|  | Compacting |  |
|  | 8B-1. Was the maximum density of the mixture determined according to AASHTO T 180, method D? (301.05) |  |
|  | 8B-2. Was each layer rolled full width, from the sides to the center, parallel to the centerline of the road? (301.05) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. | Compacting (Continued) |  |
|  | 8B-3. Were the areas along curbs, headers, walls and all places not accessible to the roller compacted with approved tampers or compactors? (301.05) |  |
|  | 8B-4. Was each layer compacted to at least 95 percent of maximum density, and moisture content determined by AASHTO T 310? (301.05) |  |
|  | Surface Tolerance |  |
|  | 9. If grade-finishing stakes were required, was the surface finished to within +or- 0.05feet from staked line and grade elevation? (301.06) |  |
|  | 10. If grade-finishing stakes were not required, was the surface shaped to the required template and the surface checked with a 10-foot straightedge? (301.06) |  |
|  | 11. Were defective areas corrected by loosening the material, adding or removing material, reshaping and compacting? (301.06) |  |
|  | Maintenance |  |
|  | 12. Was the aggregate course maintained to correct line, grade, and cross-section until placement of the next course? (301.07) |  |
|  | 13. Were all defects corrected according to Subsection 301.06? (301.07) |  |
|  | 14. |  |
|  | 15. |  |
|  | 16. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: