US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 303 Road Reconditioning

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Ditch Reconditioning |  |
|  | 1. Was all slide material, sediment, vegetation and other debris removed from the existing ditches and culvert inlets and outlets? (303.03) |  |
|  | 2. Were ditches and culvert inlets and outlets reshaped to achieve positive drainage and a uniform ditch width, depth and grade? (303.03) |  |
|  | 3. Was waste disposed of according to subsection 204.14? (303.03) |  |
|  | Shoulder Reconditioning |  |
|  | 4. Were all soft and unstable areas repaired according to subsection 204.07? (303.04) |  |
|  | 5. Was all slide material, vegetation and other debris removed from existing shoulders including shoulders of parking areas, turnouts and other widened areas? (303.04) |  |
|  | 6. Were shoulders reshaped and waste disposed of according to subsection 204.14? (303.04) |  |
|  | Roadbed Reconditioning |  |
|  | 7. Were all soft and unstable areas repaired according to subsection 204.07? (303.05) |  |
|  | 8. Was all organic, deleterious or oversized material larger than 6 inches removed from the top 6 inches of subgrade? (303.05) |  |
|  | 9. Was waste disposed of according to subsection 204.14? (303.05) |  |
|  | 10. Was surface scarified to a 6-inch depth, did surface irregularities get removed and did the surface get reshaped to a uniform shape? (303.05) |  |
|  | 11. Were the earth surfaces reshaped to within 0.05 feet and rock surfaces to within 0.10 feet of the required line, cross-section and grade? (303.05) |  |
|  | 12. Was area compacted according to subsection 204.11? (303.05) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Aggregate Surface Reconditioning |  |
|  | 13. Were all soft and unstable areas repaired to the full depth of the aggregate surface according to subsection 204.07? (303.06) |  |
|  | 14. Was the surface scarified to the depth of the aggregate surface or to depth of 8 inches, whichever was less, and surface irregularities removed? (303.06) |  |
|  | 15. Was the entire surface reshaped, finished and compacted according to subsection 308? (303.06) |  |
|  | Roadway Reconditioning |  |
|  | 16. Was all the applicable work described in subsections 303.03 through 303.06 performed? (303.07) |  |
|  | Pulverizing |  |
|  | 17. Was the surface scarified to the designated depth and width? (303.08) |  |
|  | 18. Was all material pulverized to a size one and one half times the maximum sized aggregate or to 1 1/2 inches, whichever was greater? (303.08) |  |
|  | 19. Was the material mixed, spread, compacted and finished according to section 301? (303.08) |  |
|  | 20. |  |
|  | 21. |  |
|  | 22. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: