US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 409 Asphalt Surface Treatment

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Composition |  |
|  | 1A. Was the required information and samples submitted for approval at least 21 days before placement per Subsection 409.03 paragraphs (a) thru (e)? (409.03) |  |
|  | Equipment |  |
|  | 1B. Was equipment furnished per Subsection 409.04(a)(b)(c)(d) and (e)? (409.04) |  |
|  | Surface Preparation (existing asphalt surfaces) |  |
|  | 2. Was the existing asphalt surface dry? (409.05) |  |
|  | 3. Was all loose dirt and other objectionable material removed from the surface by an approved method immediately before placement? (409.05) |  |
|  | 4. Were seal patches fogged using a slow setting emulsion diluted with equal part water? (409.05) |  |
|  | 5. Was the diluted emulsion applied at a rate of 0.17 gallons per square yard? (409.05) |  |
|  | Surface Preparation |  |
|  | 6. Was the existing aggregate surface primed according to Section 411? (409.05) |  |
|  | 7. Was the primed surface allowed to cure for at least 5 days for cutback asphalt or 24 hours for asphalt emulsions? (409.05) |  |
|  | 8. Was the primed surface dry for surface treatments using asphalt binder or slightly damp for surface treatments using emulsified asphalt? (409.05) |  |
|  | Weather Limitations |  |
|  | 9. Were surface treatments with aggregate applied only when ambient air and surface temperatures were above 60F and rising, when the weather was not foggy or rainy, and no rain for at least 24 hours after application? (409.06) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 10A. Were fog seals applied only when the ambient air and surface temperatures were above 50F and rising, when the weather was not foggy or rainy, and no rain for at least 24 hours after application? (409.06) |  |
|  | 10B. Were all surface treatment applications completed 2 hours before sunset? (409.06) |  |
|  | Production Start-up Procedures for Surface Treatments |  |
|  | 10C.Were all requirements for Production Start-up Procedures followed per Subsection 409.07? (409.07) |  |
|  | Asphalt Application |  |
|  | 11. Was the asphalt distributor spray bar height, nozzle angle, and pump pressure calibrated and was longitudinal and transverse spread rates checked weekly according to ASTM D 2995? (409.08) |  |
|  | 12. If more than one asphalt distributor was used, was each one calibrated? (409.08) |  |
|  | 13. Was the length of the spread no more than could be covered with aggregate immediately after application? (409.08) |  |
|  | 14. Were nearby objects protected to prevent spattering or marring? (409.08) |  |
|  | 15. Was building paper spread on the surface for a sufficient distance from the beginning and end of each application so the flow through the distributor nozzles could start and stop on the paper? (409.08) |  |
|  | 16. Did the CO approve the exact application rate, temperature, and area to be treated before application? (409.08) |  |
|  | 17. Was the asphalt applied uniformly with an asphalt distributor? (409.08) |  |
|  | 18. Did the distributor move forward at the proper application speed at the time the spray bar was opened? (409.08) |  |
|  | 19. Was care used not to apply excess asphalt at the junction of spreads? (409.08) |  |
|  | 20. Were skipped areas or deficiencies corrected? (409.08) |  |
|  | 21. Was the paper and other materials that were used disposed of properly? (409.08) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Aggregate Application |  |
|  | 22A. Was the aggregate surface dry if using asphalt binder, or was the aggregate surface moist if using emulsified asphalt? (409.09) |  |
|  | 22. Did the CO approve the exact application rate and area to be treated before application? (409.09) |  |
|  | 23. Was the aggregate applied uniformly with an aggregate spreader immediately after the asphalt was applied? (409.09) |  |
|  | 24. Was the aggregate spreader operated so the asphalt was covered with the aggregate before wheels passed over it? (409.09) |  |
|  | 25. If part-width construction; was an uncovered strip of sprayed asphalt approximately 6 inches wide left to permit an overlap of asphalt material? (409.09) |  |
|  | 26. Were excesses and deficiencies immediately corrected by brooming or by the addition of or removal of aggregate until a uniform texture was achieved? (409.09) |  |
|  | 27. If precoated aggregates were used was the aggregate uniformly coated with 1.0 to 2.0 percent residual asphalt by weight of aggregate? (409.09) |  |
|  | 28. Was the flow qualities of the precoated aggregate maintained, so it was satisfactorily spread with an aggregate spreader? (409.09) |  |
|  | 29. Were rollers operated at a maximum speed of 5 miles per hour? (409.09) |  |
|  | 30. Was aggregate displaced by picking-up or sticking of material to the tire surface not allowed? (409.09) |  |
|  | 31. Was the surface sufficiently rolled to uniformly, thoroughly bonding the aggregate over the full width? (409.09) |  |
|  | 32. Was the rolling completed within 1 hour after the asphalt was applied to the surface? (409.09) |  |
|  | Fog Seal |  |
|  | 33. Was the fog seal applied per the requirements of Subsection 409.10? (409.10) |  |
|  | Single-Course Surface Treatment |  |
|  | 34. Was the Single-Course Surface Treatment applied per the requirements of Subsection 409.11 and Table 409-1? (409.11) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Double- and Triple-Course Treatments |  |
|  | 35. Was the Double- and/or Triple-Course Treatments applied per the requirements of Subsection 409.12 and Table 409-3? (409.12) |  |
|  | 36. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: