US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 411 Asphalt Prime Coat

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Equipment |  |
|  | 1. Did equipment used conform to Subsection 409.04? (411.03) |  |
|  | Surface Preparation |  |
|  | 2. Was the surface to be primed prepared according to Subsection 301.05 and 301.06? (411.04) |  |
|  | Weather Limitations |  |
|  | 3. Was the prime coat applied on a dry or slightly damp surface when the air temperature in the shade and the pavement surface temperature were at least 50F and rising and when the weather was not foggy or rainy? (411.05) |  |
|  | Asphalt Application |  |
|  | 4. When required was a light spray of water applied to the surface before applying the prime coat? (411.06) |  |
|  | 5. Was asphalt applied according to Subsection 409.08 at a rate of 0.10 to 0.50 gallons per square yard for optimum penetration? (411.06) |  |
|  | 6. If emulsified asphalt that was not formulated was used as a penetrating prime coat material, was the roadway surface dampened and scarified 1 to 2 inches deep? (411.06) |  |
|  | 7. If required, was the slow-setting emulsified asphalt diluted by adding an equal amount of water? (411.06) |  |
|  | 8. Was the emulsified asphalt applied at a rate of 0.10 to 0.30 gallons per square yard? (411.06) |  |
|  | 9. Was the material immediately processed, respread, and compacted? (411.06) |  |
|  | 10. Were surfaces primed with emulsified asphalt allowed to cure for not less than 24 hours and surfaces primed with cut-back asphalt for not less than 3 days before covering with the next course? (411.06) |  |
|  | 11. Until the next course was placed, was the primed surface maintained and kept free of corrugations caused by broom dragging? (411.06) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 12. If traffic was routed over a primed surface before the asphalt material had been completely absorbed, or to minimize damage by rain, was blotter spread to cover the unabsorbed asphalt? (411.06) |  |
|  | 13. Was the excess blotter removed as soon as practicable after excess asphalt was absorbed? (411.06) |  |
|  | 14. Was all dirt or other deleterious materials removed and were all damaged areas repaired before placing the next course? (411.06) |  |
|  | 15. |  |
|  | 16. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: