US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 416 Continuous Cold Recycled Asphalt Base Course

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Submittals |  |
|  | 1A. If type A compaction was designated, was the requirements of paragraphs (1) thru (4) of subsection 416.03 submitted at least 28 days before recycling began? (416.03) |  |
|  | 1B. Was the name, qualifications, and references of a competent person with extensive recycling experience submitted at least 14 days before recycling began? (416.03) |  |
|  | 1C. Was the proposed equipment, manpower, and sequence of operations submitted at least 14 days before recycling began? (416.03) |  |
|  | Surface Preparation |  |
|  | 1. Was the existing surface cleaned of all loose material, dirt or other deleterious substances by approved methods? (416.04) |  |
|  | 2. Were all the areas cleared, grubbed and all vegetation and debris removed within 2 feet of the pavement to be recycled? (416.04) |  |
|  | Weather Limitations |  |
|  | 3. Was the emulsified asphalt pavement placed on a dry, unfrozen surface only when the air temperature in the shade was above 50F and the pavement surface temperature was above 40F? (416.05) |  |
|  | 4. Within 24 hours following the placement of the mix did fog, showers, rain, frost or temperatures below 35F occurred? (416.05) |  |
|  | Pavement Milling |  |
|  | 5. Did the equipment used meet the following criteria: (a) Self-propelled, (b) Equipped with automatic depth controls, (c) Capable of maintaining the required cutting depth, (d) Capable of milling to the required depth in a single pass of half the existing pavement width or one lane width, whichever was greater and (e) Capable of screening and crushing material? (416.06) |  |
|  | 6. Was the underlying material disturbed? (416.06) |  |
|  | 7. Were all oversized particles reduced to a maximum particle size of 1 inch? (416.06) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Mixing and Proportioning |  |
|  | 8. Was the proper equipment used and monitoring done continuously per Subsection 416.07? (416.07) |  |
|  | Spreading, Finishing, and Compacting |  |
|  | 9. Was a paver conforming to Subsection 401.05 used that was capable of picking up the entire windrow and feeding it into the paver hoper, and the screed was not heated? (416.08) |  |
|  | 10. Was the recycled mix placed, spread and struck off to the required line, grade, and elevation? (416.08) |  |
|  | 11. Was initial compacting; pavement smoothness, final compaction and curing performed according to Subsection 416.08(a), (b), (c) and (d)? (416.08) |  |
|  | Fog Seal |  |
|  | 12. If there is a pay item for fog seal in the bid schedule and if directed by the CO, was a fog seal placed on the surface of the recycled base after final compaction? (416.09) |  |
|  | 13. If fog seal was used was it diluted 50 percent by volume with water and applied at a rate of 0.04 to 0.20 gallons per square yard? (416.09) |  |
|  | 14. Was blotter placed according Section 411? (416.09) |  |
|  | 15. |  |
|  | 16. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: