US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 552 Structural Concrete

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Composition (Concrete Mix Design) |  |
|  | 1. Was a concrete mix design submitted in accordance with Subsection 552.03? (552.03) |  |
|  | 2. Does the submitted concrete mix design meet the requirements of Subsection 552.03? (552.03) |  |
|  | Storage and Handling of Material |  |
|  | 3. Were all materials stored and handled in a manner that prevents segregation, contamination, or other harmful effects? (552.04) |  |
|  | 4. Was cement and fly ash containing evidence of moisture contamination used? (552.04) |  |
|  | 5. Was aggregate stored and handled in a manner that ensured uniform moisture content at the time of batching? (552.04) |  |
|  | Measuring Material |  |
|  | 6. Was the concrete batched according to the approved mix design and the following tolerances: Cement  1 percent, Water  1 percent, Aggregate  2 percent, and Additive  3 percent? (552.05) |  |
|  | Batching Plant, Mixers and Agitators |  |
|  | 7. If a batching plant, mixer and agitator was used did it conform to AASHTO M 157? (552.06) |  |
|  | 8. Did the continuous volumetric mixing equipment conform to AASHTO M 241? (552.06) |  |
|  | Mixing |  |
|  | 9. Was all mixing equipment (mix plant or truck) operated within manufacturer’s recommended capacity? (552.07) |  |
|  | 10. Was the concrete produced, of a uniform consistency? (552.07) |  |
|  | 11. If concrete was produced in a Central-mix plant was it according to Subsection 552.07(a)? (552.07a) |  |
|  | 12. If concrete was mixed in a truck were any sections of the blades worn 1 inch or more below the original manufactured height? (552.07b) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 13. If concrete was mixed in a truck did the mixers and agitators in the mixing drum have accumulated hard concrete or mortar on them? (552.07b) |  |
|  | 14. If concrete was mixed in a truck, were admixtures added to the mix water before or during mixing? (552.07b) |  |
|  | 15. If concrete was mixed in a truck was the batch charged into the drum so a portion of the mixing water entered in advance of the cement? (552.07b) |  |
|  | 16. If concrete was mixed in a truck was each batch mixed according to AASHTO M 157? (552.07b) |  |
|  | Delivery |  |
|  | 17. Was concrete delivered according to Subsection 552.08 and Table 552-4? (552.08) |  |
|  | Quality Control of Mix |  |
|  | 18. Was the quality control of mix performed according to Subsection 552.09? (552.09) |  |
|  | Temperature and Weather Conditions |  |
|  | 19. Was the temperature of the concrete mixture just before placement between 50 and 90F, except for bridge decks the mixture should be between 50 and 80F? (552.10) |  |
|  | 20. Was cold weather placement done according to Subsection 552.10(a) and for hot weather placement according to Subsection 552.10(b)? (552.10a,b) |  |
|  | 21. When placing concrete in bridge decks or other exposed slabs, was the expected evaporation rate less than 0.1 pound per square foot per hour as determined by figure 552-1? (552.10) |  |
|  | 22. If necessary was one or more of the following actions taken: (1) Construct windbreaks or enclosures to reduce wind velocity throughout the area of placement; (2) Use of fog sprayers upwind of placement to increase relative humidity; (3) Reduce the temperature of the concrete according to Subsection 552.10(b)? (552.10c) |  |
|  | 23. Was the concrete protected from rain at all times during and immediately after placement? (552.10d) |  |
|  | Handling and Placing Concrete |  |
|  | 24. Was the handling and placing of the concrete done according Subsection 552.11? (552.11) |  |
|  | Construction Joints |  |
|  | 25. Were construction joints provided at locations shown on the plans? (552.12) |  |
|  | 26. If additional construction joints were added was written approval provided? (552.12) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 27. Was reinforcing steel extended uninterrupted through construction joints? (552.12) |  |
|  | 28. Were lap splices or mechanical splices embedded within the concrete and not in a construction joint? (552.12) |  |
|  | 29. Dowels were not used? (552.12) |  |
|  | 30. Were gauge strips placed at horizontal construction joints inside the forms along all exposed faces to produce straight joint lines? (552.12) |  |
|  | 31. If a joint was between two fresh concrete placements, was the first placement rough floated to thoroughly consolidate the surface and leave the joint surface in a roughened condition? (552.12) |  |
|  | 32. Was the joint surface kept saturated? (552.12) |  |
|  | 33. Immediately before placing new concrete, were the forms drawn tightly against previously placed concrete and, where accessible, was the joint surface thoroughly coated with a very thin coating of cement mortar? (552.12) |  |
|  | 34. If a joint was between existing concrete and a new placement, was the existing concrete cleaned by abrasive blasting or other approved method to remove all laitance and foreign material, to expose clean aggregate, and to roughen the joint surface? (552.12) |  |
|  | 35. Before concrete was placed, was an approved bonding product applied to the joint surface according to the manufacture’s recommendation? (552.12) |  |
|  | Expansion and Contraction Joints |  |
|  | 36. Were expansion and contraction joints formed according to Subsection 553.13? (552.13) |  |
|  | Finishing Plastic Concrete |  |
|  | 37. Was the finishing of plastic concrete performed according to Subsection 552.14? (552.14) |  |
|  | Curing Concrete |  |
|  | 38. Was curing begun immediately after the free surface water was evaporated and the finishing was completed? (552.15) |  |
|  | 39. If the surface of the concrete began to dry before the selected cure method was implemented was the concrete surface kept moist using fog spray, without damaging the surface? (552.15) |  |
|  | 40. Were surfaces to be rubbed kept moist after forms were removed and was the surface cured immediately following the first rub? (552.15) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 41. Were the top surfaces of bridge decks cured using the liquid membrane curing compound method combined with the water method and if so was the liquid membrane curing compound applied immediately after finishing and the water cure applied within 4 hours after finishing? (552.15) |  |
|  | 42. Was all concrete cured uninterrupted for at least 7 days, unless pozzolans were used in excess of 10 percent by mass of the hydraulic cement and then cured uninterrupted for at least 10 days? (552.15) |  |
|  | 43. If the following methods of curing were used were they used according to Subsection 552.15 (a) forms in place, (b) water method or (c) liquid membrane curing compound? (552.15) |  |
|  | 44. If the steam-curing method was used, was it used according to Subsection 552.15(d)? (552.15) |  |
|  | Finishing Formed Concrete Surfaces |  |
|  | 45. Were all rock pockets or honeycombed concrete removed and replaced or repaired? (552.16) |  |
|  | 46. Were all sound, formed concrete surfaces finished according to Subsections 552.16 (a), (b), (c), (d), (e), (f) or (g) depending of the finish required? (552.16) |  |
|  | Concrete Anchorage Devices |  |
|  | 47. Were concrete anchorage devices tested and installed according to Subsection 522.17? (552.17) |  |
|  | Loads on New Concrete Structures |  |
|  | 48. Was traffic kept off of concrete bridge decks until all deck concrete had attained the design compressive strength and had been in place for at least 14 days or longer? (552.18) |  |
|  | 49. Construction loads of 4000 pounds or more were not placed on the deck until after the concrete had cured at least 7 days and the concrete in the entire span had attained a compressive strength of at least 70 percent of the specified design strength? (552.18) |  |
|  | 50. For precast concrete multi-beam sections, vehicles were not allowed on any span before the grout attained strength of 3000 pounds per square inch and tie rods had been tightened? (552.18) |  |
|  | 51. For post-tensioned concrete structures, vehicles were not allowed over 4500 pounds on any span before the prestressing steel for that span was tensioned, grouted, and cured, the grout obtained a strength of 3000 pounds per square inch, and the tie rods tightened? (552.18) |  |
|  | 52. Were vehicles’ weighing less than 4500 pounds permitted on a span provided the mass of the vehicle was included in the falsework design? (552.18) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: