US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 564 Bearing Devices

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | General |
|  | 1. Were drawings prepared and submitted for the bearings according to Subsection 104.03 and Section 18 of the AASHTO Standard Specifications for Highway Bridges Division II, Volume II? (564.03a) |
|  | 2. On the drawings were all details of the bearings shown including the material proposed for use? (564.03a) |
|  | 3. Was approval of the drawings obtained before beginning fabrication? (564.03a) |
|  | 4. Were bearings fabricated according to Section 18 of the AASHTO Standard Specifications for Highway ridges Division II, Volume II? (564.03b) |
|  | 5. Did the surface finish of bearing components that were in contact with each other or with concrete, but not embedded in concrete, conform to Subsection 555.08(e)? (564.03b) |
|  | 6. Were bearing assemblies preassembled in the shop and checked for proper completeness and geometry? (564.03b) |
|  | 7. Were steel bearing components and anchor bolts galvanized? (564.03b) |
|  | 8. Were stainless steel bearing components or anchor bolts galvanized? (564.03b) |
|  | 9. Before shipment from the manufacturer, were the bearing components clearly identified and marked on top, the location and orientation in the structure? (564.03c) |
|  | 10. Were the bearings securely bolted, strapped or otherwise fastened to prevent any relative movement? (564.03c) |
|  | 11. Were the bearings packaged so they were protected from damage due to shipping, handling, weather, or other hazards? (564.03c) |
|  | 12. Were bearing assemblies dismantled at the site only for the purpose of inspection or installation? (564.03c) |
| Conformance | CHECKS (characteristics) |
| Yes No N.A. |  |
|  | 13. Were all bearing devices and components stored at the work site in a location that provided protection from environmental and physical damage? (564.03c) |
|  | 14A. Were the bearings cleaned of all deleterious substances? (564.03d) |

|  | 14B. Were the bearings installed to the positions shown on the drawings? (564.03d) |  |
| --- | --- | --- |
|  | 15. Were bearings and bearing components set to the dimensions shown on the drawings or as prescribed by the manufacturer? (564.03d) |  |
|  | 16. Were the bearings adjusted according to the manufacturer’s instructions to compensate for installation temperature and future movements of the bridge? (564.03d) |  |
|  | 17. Were bearings set level at the exact elevation and position? (564.03d) |  |
|  | 18. Was full and even bearing on all external bearing contact surfaces provided? (564.03d) |  |
|  | 19. If bearing surfaces were at improper elevations, not level, or if bearings were not otherwise set properly, was the CO notified and a written proposal to modify the installation submitted for approval? (564.03d) |  |
|  | 20. Were metallic bearing assemblies, not embedded in concrete, bedded on concrete with an approved filler or fabric material? (564.03d) |  |
|  | 21. Were Elastomeric bearing pads directly set on properly prepared concrete surfaces without bedding material? (564.03d) |  |
|  | 22. Were bearing surfaces seated directly on steel machined to provide a level and planar surface upon which to place the bearing? (564.03d) |  |
|  | Elastomeric Bearings |  |
|  | 23A. Were elastomeric bearings more than ½ inch thick reinforced with laminates every ½ inch through the entire thickness? (564.04) |  |
|  | 23. Were Elastomeric bearings fabricated according to AASHTO M 251? (564.04) |  |
|  | 24. Did material used, meet the flash tolerance, finish, and appearance requirements of the Rubber Handbook as published by the Rubber Manufacturer’s Association Incorporated, RMA F 3 and T.063 for molded bearings and RMA F2 for extruded bearings? (564.04) |  |
|  | 25. Was compliance with AASHTO M 251, level I acceptance criteria met? (564.04) |  |
|  | 26. Was each reinforced bearing marked with indelible ink or flexible paint and the information include the order number, lot number, bearing identification number, and elastomer type and grade number? (564.04) |  |
|  | 27A. Were markings placed on a face that was visible after erection of the bridge? (564.04) |  |
|  | 27B. Was a list of all individual bearing members furnished? (564.04) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 28. Were bearings placed on a level surface and any misalignments in the support corrected to form a level surface? (564.04) |  |
|  | 29. Steel girders or base plates were not welded to the exterior plates of the bearing unless there was more then 1½ inches of steel between the weld and elastomeris ? (564.04) |  |
|  | 30. Was the elastomer or elastomer bond exposed to instantaneous temperatures greater than 400F? (564.04) |  |
|  | Rocker, Roller, and Sliding Bearings |  |
|  | 31. If TFE coatings were required did they conform to Subsection 564.07? (564.05) |  |
|  | 32. Were rocker, roller and sliding bearings fabricated according to the details shown on the plans and in section 555? (564.05) |  |
|  | 33. Were bearings fabricated according to the standard practice in modern commercial shops? (564.05) |  |
|  | 34. Were burrs, rough and sharp edges and other flaws removed? (564.05) |  |
|  | 35. Were rocker, roller, and other bearings relieved of the stress’ that are built up by welding sections of plate together before boring, straightening, or finished machining? (564.05) |  |
|  | 36. Were all contact surfaces thoroughly coated with oil and graphite just before placing roller bearings? (564.05) |  |
|  | 37. Were rocker, roller and sliding bearings installed so they are vertical at the specified mean temperature after release of falsework and after any shortening due to prestressing forces? (564.05) |  |
|  | 38. Were variations from mean temperature of the supported span at time of installation and any other anticipated changes in length of the supported span taken into account? (564.05) |  |
|  | 39. Did the superstructure have full and free movement at movable bearings? (564.05) |  |
|  | 40. Were the cylindrical bearings positioned so that their axes of rotation aligned and coincided with the axis of rotation of the superstructure? (564.05) |  |
|  | Masonry, Sole, and Shim Plates for Bearings |  |
|  | 41. Were metal plates used in masonry, sole, and shim plates conforming to AASHTO M 270, grade 36? (564.06) |  |
|  | 42. Was steel fabricated and finished according to Section 555? (564.06) |  |
|  | 43. Were holes in bearing plates formed by drilling, punching, or accurately controlled oxygen cutting and all burrs removed by grinding? (564.06) |  |
|  | 44. Were bearing plates accurately set in level position as shown on the drawings and did they provide a uniform bearing over the bearing contact area? (564.06) |  |
|  | 45. When plates were embedded in concrete, were provision made to keep them in the correct position as the concrete was placed? (564.06) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Tetrafluoroethylene (TFE) Surfaces for Bearings |  |
|  | 46. Was TFE material furnished that was factory-bonded, mechanically connected, or recessed into the backup material as shown on the plans? (564.07) |  |
|  | 47. Was the fabric containing TFE, bonded or mechanically attached to a rigid substrate? (564.07) |  |
|  | 48. Was a fabric capable of carrying unit loads of 10,000 pounds per square inch without cold flow used? (564.07) |  |
|  | 49. Was a fabric-substrate bond used that was capable of withstanding, without delamination, a shear force equal to 10 percent of the perpendicular or normal application loading plus any other bearing shear forces? (564.07) |  |
|  | 50. Was compliance determined using approved test methods and procedures according to Section 18, Subsection 18.8.3, AASHTO Standard Specifications for Highway Bridges Division II, Volume II? (564.07) |  |
|  | 51. If the test facility did not permit testing completed bearings, were extra bearings manufactured and prepared samples of at least 100-kip capacity at normal working stresses? (564.07) |  |
|  | 52. Was static and dynamic coefficient of friction determined at first movement of the test bearing at a sliding speed of less than 1 inch per minute? (564.07) |  |
|  | 53. Did the coefficient of friction exceed the coefficient of friction as specified in Table 564-1 or by the manufacturer? (564.07) |  |
|  | 54. Was a listing of all individual bearing numbers furnished? (564.07) |  |
|  | Anchor bolts |  |
|  | 55. Were swedge or thread anchor bolts furnished that conform to ASTM A 307 or as shown on the plans or specified in the contract? (564.08) |  |
|  | 56. Were anchor bolts preset before placement of the concrete or installed in drilled holes after placement of the concrete? (564.08) |  |
|  | 58. If bolts were installed after concrete placement, were they secured in the drilled holes with non-shrink cement grout or an approved chemical adhesive? (564.08) |  |
|  | 59. If non-shrink cement grout was used, were holes drilled 1 inch in diameter greater than the bolt? (564.08) |  |
|  | 60. If chemical adhesives were used, was the adhesive manufacturer’s recommendations followed for hole-diameter? (564.08) |  |
|  | 61A. Were bolt locations for superstructure adjusted for temperature as required? (564.08) |  |
|  | 61. Free movement of the superstructure was not restricted at movable bearings by anchor bolts or nuts? (564.08) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Bedding of Masonry Plates |  |
|  | 62. If required by the contract was filler or fabric placed as bedding material under masonry plates? (564.09) |  |
|  | 63. Was the type of filler or fabric specified used and installed to provide full bearing on contact areas? (564.09) |  |
|  | 64. Was the contact surfaces of the concrete and steel thoroughly cleaned immediately before placing the bedding material and installing bearings or masonry plates? (564.09) |  |
|  | 65. If bedding materials were not specified, did the materials comply with AASHTO 18.4.10 as directed by the CO? (564.09) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: