US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 601 Minor Concrete Structures

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Concrete Composition |
|  | 1. Before batching concrete, was a proposed concrete proportions submitted for approval on Form FHWA 1606 Minor Concrete Mix Design Trial Batch Summary or other approved form per subsection 601.03 (a) thru (i)? (601.03) |
|  | General |
|  | 2. Was excavation and backfill done according to Section 209? (601.04) |
|  | 3. If concrete was cracked, spalling or scaling, was it removed to the nearest joint? (601.04) |
|  | 4. Were forms designed and constructed free of bulging and warping, and removed without damaging the concrete? (601.04) |
|  | 5. If concrete contained a retarding admixture, fly ash, or other pozzolans replacement for cement, were the forms designed for a lateral pressure equal to that exerted by a fluid weighing 150 pounds per cubic foot? (601.04) |
|  | 6. Were forms clean and coated with a form release agent or form oil before placing concrete? (601.04) |
|  | 7. Was reinforcing steel placed and fastened according to Subsection 554.08? (601.04) |
|  | Placing concrete |
|  | 8. Was concrete placed according to Subsection 552.10? (601.05) |
|  | 9. Were forms and foundation moistened immediately before placing concrete? (601.05) |
|  | 10. Was the concrete discharged within the time limit shown in Table 552-4? (601.05) |
|  | 11. Was concrete placed as to avoid segregation of material? (601.05) |
|  | 12. Was concrete consolidated with vibrators according to Subsection 552.11(d)? (601.05) |
|  | 13. Aluminum pipe was not used for transporting or placing concrete? (601.05) |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | 14. The intervals between deliveries of batches for a single pour on a structure did not exceed 30 minutes? (601.05) |  |
|  | 15. Water was not applied to plastic concrete surfaces during finishing operation? (601.05) |  |
|  | Curing concrete |  |
|  | 16. Was concrete cured a minimum of 7 days, unless high early strength cement was used, then cure for a minimum 3 days? (601.06) |  |
|  | 17. Was concrete cured according to Subsection 552.15? (601.06) |  |
|  | 18. Were exposed concrete surfaces finished according to Subsection 552.16(a) or (b), as applicable? (601.06) |  |
|  | 19. |  |
|  | 20. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % ConformanceYesYes + No(100) (100) % |

Comments: