US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 602 Culverts and Drains

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. Was culvert pipe furnished with a wall thickness not less than that shown on the plans or determined from the fill-height tables included in the plans? (602.03) |  |
|  | 2. Were the same materials and coatings used on all contiguous pipe sections and special sections such as elbows and branch connections? (602.03) |  |
|  | 3. For culvert extensions, was the same material as the existing culvert used? (602.03) |  |
|  | 4. Was the final location, skew, length, elevations and grade determined according to Subsection 152.03(g)? (602.03) |  |
|  | 5. The culvert material was not ordered until the CO accepted the final structure size, length, and alignment? (602.03) |  |
|  | 6. Was excavation and backfill performed according to Section 209? (602.03) |  |
|  | Laying Concrete Pipe and Precast Concrete Box Culverts |  |
|  | 7. Was the pipe or culvert started at the lower end and laid with the bell or groove end upgrade? (602.04) |  |
|  | 8. Were all sections fully joined? (602.04) |  |
|  | 9. Was circular pipe with elliptical reinforcement placed with the minor axis of the reinforcement in a vertical position? (602.04) |  |
|  | 10. Were mortar joints constructed according to Subsection 602.04(a)? (602.04a) |  |
|  | 11. Were gasketed joints constructed according to Subsection 602.04(b)? (602.04b) |  |
|  | Laying Metal Pipe |  |
|  | 12. Were pipes laid with outside laps of circumferential joints upgrade and longitudinal laps positioned other than in the invert? (602.05) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 13. Were elongated pipes placed with major axis within 5 degrees of vertical? (602.05) |  |
|  | 14. Were pipe sections joined together with soil tight bell and spigot joints or coupling bands according to AASHTO M 36 or M 196? (602.05) |  |
|  | 15. Was the use of bell and spigot joints limited to slopes of 10 percent or less? (602.05) |  |
|  | 16. Was the use of coupling bands with projections (dimples) limited to attaching prefabricated flared end sections? (602.05) |  |
|  | 17. If aluminum alloys were in contact with other metals, were the contacting surfaces coated with asphalt mastic or a preapproved impregnated caulking compound? (602.05) |  |
|  | Laying Plastic Pipe |  |
|  | 18. Was the plastic pipe laid according to the pipe manufacturer’s recommendations? (602.06) |  |
|  | 19. Were soil-tight and spigot joints for plastic pipe culverts provided? (602.06) |  |
|  | 20. If plastic end sections were used, were they reinforced and stiffened such that inward buckling during construction was less than 3/8 inch with a 100-pound force? (602.06) |  |
|  | Laying Slotted Drain Pipe |  |
|  | 21. Were the sections joined together with coupling bands? (602.07) |  |
|  | 22. Were the slots covered with roofing paper or other approved covering during backfilling and paving to keep material out of the pipe? (602.07) |  |
|  | 23. Was lean concrete used to backfill? (602.07) |  |
|  | 24. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: