US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 603 Structural Plate Structures

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. Was excavation and backfill completed according to Section 209? (603.03) |  |
|  | Erecting |  |
|  | 2. Was a copy of the manufacturer’s assembly instructions provided before assembly? (603.04) |  |
|  | 3. Did the instructions show the position of each plate and the assembly order? (603.04) |  |
|  | 4. Were the structural plates assembled according to the manufacturer’s instructions? (603.04) |  |
|  | 5. Was care exercised in the use of drift pins and pry bars to prevent damage to the structural plate and its coating? (603.04) |  |
|  | 6. Did the plates have a proper fit-up? (603.04) |  |
|  | 7. If aluminum alloys came in contact with other types of metal, were the contacting surfaces coated according to Subsection 602.05? (603.04) |  |
|  | 8. Were steel bolts on steel plates torqued to a minimum of 100 foot-pounds and a maximum of 300 foot-pounds? (603.04) |  |
|  | 9. Were steel bolts and aluminum bolts on 0.1-inch thick aluminum plates torqued to a minimum of 90 foot-pounds and a maximum of 115 foot-pounds? (603.04) |  |
|  | 10. Were steel bolt and aluminum bolts on 0.1-inch thick and heavier aluminum plates torqued to a minimum of 90 foot-pounds and a maximum of 115 foot-pounds? (603.04) |  |
|  | Erecting (for long-span structures) |  |
|  | 11. Were the longitudinal seams tighten when the plates were assembled unless the plates were held in shape by cable, struts, or backfill? (603.04a) |  |
|  | 12. Were plates circumferentially properly aligned to avoid permanent distortion from the design shape? (603.04a) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 13. Before backfilling, a 2 percent variation from design shape was not exceeded? (603.04a) |  |
|  | 14. Was the shape of the structure distorted by operating equipment over or near it? (603.04b) |  |
|  | 15. Was suitable survey control provided on the structure to check structure movement? (603.04c) |  |
|  | 16. Was the deflection movement checked and controlled during the entire backfilling operation? (603.04d) |  |
|  | 17. Was the manufacturer’s recommended limits exceeded during backfilling? (603.04d) |  |
|  | 18. Was a manufacture’s representative provided to monitor the erection and backfilling of the structure? (603.04e) |  |
|  | 19. |  |
|  | 20. |  |
|  | 21. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: