US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 605 Underdrains, Sheet Drains, and Pavement Edge Drains

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | General |
|  | 1A. Was the same material and coating used on all contiguous drain sections, extensions, elbows, branch connections, and other special sections? (605.03) |
|  | 1B. Was the location and length determined in the field and the design submitted for approval? (605.03) |
|  | 2. The drain material was not installed before the CO had accepted the final location and length? (605.03) |
|  | 3. Was excavation and backfill completed according to Section 209? (605.03) |
|  | 4. If geotextile or geocomposite was used, were the trench surfaces smoothed by removing all projections that may have damaged the geotextile or geocomposite? (605.03) |
|  | 5. If the geotextile or geocomposite was damaged during installation, was the damaged area replaced with good material? (605.03) |
|  | 6. If repairs to geocomposites were made were they according to the manufacturer’s recommendations? (605.03) |
|  | 7. Soil or other foreign material was not permitted to enter the drain system? (605.03) |
|  | 8. Was the upgrade end of installations plugged? (605.03) |
|  | 9. Was nonperforated pipe furnished for outlet pipe? (605.03) |
|  | 10. Was outlet pipe installed according to Section 602? (605.03) |
|  | 11. Was a screen made of 17 gage diameter galvanized wire having approximately ½ by ½ mesh openings immediately placed and secured over the outlet ends of all exposed pipes and weep holes? (605.03) |
|  | Placing Underdrain |
|  | 12. Were pipe sections joined securely with coupling fittings or bands? (605.04) |
|  | 13. Were PVC and ABS pipes joined using either a flexible elastomeric seal or solvent cement? (605.04) |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | 14. Were polyethylene pipes joined with snap-on, screw-on, or wrap around coupling bands as recommended by the manufacturer? (605.04) |
|  | 15. Were all trenches within the limits of the roadbed backfilled and compacted according to Section 209 except was granular backfill material used? (605.04) |
|  | 16. If when underdrain was placed in ditch lines, was infiltration of surface water prevented by placing material conforming to AASHTO M 145, A-4, A-5, A-6, or A–7 in the top 12 inches of the trench? (605.04) |
|  | Placing Underdrain (Standard Underdrain) |
|  | 17. When geotextile was required, was it placed with the long dimension of the geotextile parallel to the centerline of the trench? (605.04a) |
|  | 18. Was geotextile positioned without stretching, and in contact with the trench surface? (605.04a) |
|  | 19. Were the joints overlapped a minimum of 24 inches with the upstream geotextile placed over the downstream geotextile? (605.04a) |
|  | 20. Was the collector pipe placed with the perforations facing downward? (605.04a) |
|  | 21. Was granular backfill placed to a height of 12 inches above the top of the collector pipe and compacted? (605.04a) |
|  | 22. Was the collector pipe not displaced while being backfilled? (605.04a) |
|  | 23. Was the remainder of the granular backfill material placed and compacted according to Section 209? (605.04a) |
|  | 24. Was the geotextile folded over the top of the granular backfill with a minimum overlap of 12 inches? (605.04a) |
|  | Placing Underdrain (Geocomposite Underdrain) |
|  | 25. Was the geotextile extended from the bottom of the drainage core around the collector pipe? (605.04b) |
|  | 26. Were splices and installed outlet fittings constructed according to the manufacturer’s recommendations? (605.04b) |
|  | 27. Was infiltration of soil into the geocomposite core prevented? (605.04b) |
|  | 28. Was the assembled geocomposite in the trench placed with the face of the geocomposite against the inflow side of the trench? (605.04b) |
| Conformance | CHECKS (characteristics) |
| Yes No N.A. |  |
|  | 29. If the trench wall was irregular, was the trench wall smoothed or was a layer of granular backfill placed between the geocomposite and the trench wall? (605.04b) |
|  | 30. Was the drain temporarily supported against the trench wall while backfilling? (605.04b |
|  | 31. When the trench was less than 18 inches wide, was the trench backfilled using clean sand? (605.04b) |
|  | 32. Was the sand compacted by vibrating, tamping with a mechanical tamper, or flooding with water? (605.04b) |
|  | 33. When the trench was 18 inches wide or more was granular backfill or clean sand placed to a height of 12 inches above the top of the collector pipe and compacted? (605.04b) |
|  | 34. Was the remainder of the granular backfill material or clean sand placed and compacted according to Section 209? (605.04b) |
|  | Placing Geocomposite Sheet Drain |
|  | 35. Was the sheet drain placed against a mortar course less than 4 days old? (605.05) |
|  | 36. If a geocomposite was used in conjunction with a waterproof membrane, were drainage panels installed that were compatible with the membrane using methods recommended by the membrane manufacturer? (605.05) |
|  | 37. Was the geocomposite drain assembled and placed against the surface to be backfilled according to the manufacturer’s recommendation? (605.05) |
|  | 38. Were geocomposite drains spliced so the flow across the edges was continuous? (605.05) |
|  | 39. Was the geotextile overlapped a minimum of 3 inches in the direction of water flow? (605.05) |
|  | 40. For vertical splices, was the geotextile overlapped in the direction backfill proceeded? (605.05) |
|  | 41. Was the drainage core to the collector pipe or weep holes connected so the flow was continuous through the system? (605.05) |
|  | 42. Was the geotextile extended from the bottom of the drainage core around the collector pipe? (605.05) |
|  | 43. Was the underdrain backfilled with structural backfill and compacted according to Subsections 208.10 and 208.11? (605.05) |

| Yes No N.A. |  |  |
| --- | --- | --- |
|  | Placing Geocomposite Pavement Edge Drain |  |
|  | 44. Was the geocomposite pavement edge drain and outlet material assembled according to the manufacturer’s recommendations? (605.06) |  |
|  | 45. If the trench wall was irregular, was it smoothed or was a layer of clean sand placed between the geocomposite and the trench wall? (605.06) |  |
|  | 46. Was the drain supported against the trench wall while being backfilling? (605.06) |  |
|  | 47. If the trench was less then 18 inches wide, was it backfilled with clean sand? (605.06) |  |
|  | 48. Was the sand compacted with a vibrating tamper or flooding with water? (605.06) |  |
|  | 49. If the trench was wider than 18 inches, was it backfilled with granular backfill and compacted or clean sand according to Section 209. |  |
|  | 51. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: