US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 609 Curb and Gutter

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | General |
|  | 1. Was excavation and backfill completed according to Section 209? (609.03) |
|  | 2. Was the bed course material placed and compacted with at least three passes of a lightweight mechanical tamper, roller, or vibratory system? (609.03) |
|  | Stone and Precast Concrete Curb |
|  | 3A1. Was the curb material cleaned and wet just prior to setting? (609.04) |
|  | 3A2. Was the curb set in the bed course so the face and top lines were to line and grade? (609.04) |
|  | 3A3. Were the joints ½ to 1 inch wide and filled with mortar? (609.04) |
|  | 3A4. Was the first 25 feet of curb build to demonstrate the ability to build a curb meeting the above requirements? (609.04) |
|  | 3A5. Work did not continue on remaining curb until the first 25 feet test section was approved? (609.04) |
|  | 3A6. If concrete pavement was constructed contiguous to the curb, were the joints in the curb constructed directly in line with the pavement expansion joints? (609,04) |
|  | 3A7. Was the curb joint ¾ inch wide and filled with expansion joint filler of the same nominal thickness as the pavement joint? (609.04) |
|  | 3A8. Were the voids between the joint filler and the curb filled with mortar? (609.04) |
|  | Concrete Curb or Curb and Gutter (Cast-in-Place) |
|  | 3B. Was worked performed according Section 601? (609.05) |
|  | 3C. Were curved forms used for radius of 300 feet or less? (609.05a) |
|  | 4. Were forms used that extended for the full depth of the concrete? (609.05a) |
| Conformance | CHECKS (characteristics) |
| Yes No N.A. |  |
|  | 5. For contraction joints were curb sections constructed in sections of uniform 10-foot lengths? (609.05a1) |
|  | 6. Were contraction joints constructed 1/8 inch wide? (609.05a1) |
|  | 7. Were metal divider plates used? (609.05a1) |
|  | 8. Were expansion joints formed at intervals of 60 feet using ¾ inch thick preformed expansion joint filler? (609.05a2) |
|  | 9. When the curb was constructed adjacent to or on concrete pavement, did the expansion joints match the joints in the pavement? (609.05a2) |
|  | 10. Was the concrete finished smooth and even with a wood float? (609.05a) |
|  | 11. If broom finished was it parallel to the curb line according to Subsection 552.14(c)(2)? (609.05a) |
|  | 12. If exposed aggregate finish was required was it according to Subsection 552.14(c)(4)? (609.05a) |
|  | 13. Were forms left in place for 24 hours or until the concrete had set sufficiently so the forms could be removed without harming the curb? (609.05a) |
|  | Concrete Curb or Curb and Gutter (Slip-Formed) |
|  | 14. Was a self-propelled automatic curb machine or a paver with curb attachments used? (609.05b) |
|  | 15. Was a machine that was heavy enough to obtain consolidation without the machine riding above the foundation used? (609.05b) |
|  | 16. Was the concrete aggregate gradation adjusted, if necessary, to produce a curb or curb and gutter that had well-defined web marks of water on the surface? (609.05b) |
|  | 17. Were sections with craters larger than 3/16 inch or other sections determined to be damaged or defective removed and replaced? (609.05b) |
|  | 18. Surface craters and other defective sections were not repaired by plastering? (609.05b) |
|  | 19. Was concrete hardened sufficiently to permit sawing of contraction joints without causing damage to the concrete surface, according to 609.05 (a)(1)? (609.05b) |
|  | 19B. Were expansion joints constructed according to 609.05 (a)(2)? (609.05b) |
| Conformance | CHECKS (characteristics) |
| Yes No N.A. |  |
|  | Asphalt Concrete Curb |
|  | 20. Where curb was constructed on a pavement, was a tack coat placed according to Section 412 on the area under the curb? (609.06) |
|  | 21. Was asphalt concrete curb constructed according to Section 404? (609.06) |
|  | 22. Was a self-propelled automatic curb machine or a paver with curb attachments used, that was heavy enough to compact a curb without riding above the foundation? (609.06) |
|  | 23. Was the curb made uniform in texture, shape, and density? (609.06) |
|  | 24. Was curb constructed by other means only in short sections or sections with short radii? (609.06) |
|  | Resetting Stone or Precast Concrete Curb |
|  | 25. Was the curb carefully removed, cleaned and stored? (609.07) |
|  | 26. Was all lost, damaged, or destroyed curb replaced? (609.07) |
|  | 27. Was the curb reset according to Subsection 609.04? (609.07) |
|  | Wheel stops |
|  | 28. Were the wheelstops pinned into place with two 3.5-foot sections of No. 6 reinforcing steel or ¾-inch steel rod. (609.08) |
|  | 29. Were wheelstops reset in the same manor as above? (609.08) |

| Percent Conformance Calculations | YesYes + No(100)= (100)= % Conformance |
| --- | --- |

Comments: