US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 617 Guardrail

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Posts |  |
|  | 1. Were field cuts for wood posts treated with two coats of preservative that was applied with a brush or sprayer? (617.03) |  |
|  | 2. Were field cuts placed in contact with the ground? (617.03) |  |
|  | 3. If the pavement surface was within 3 feet of the guardrail were the posts installed before placing the pavement surface? (617.03) |  |
|  | 4. Were post holes punched or drilled no more than ½ inch larger than the post dimensions? (617.03) |  |
|  | 5. Were the posts driven into the pilot holes and set plumb? (617.03) |  |
|  | 6. Were the posts backfilled and compacted with acceptable material? (617.03) |  |
|  | 7. If it was not possible to maintain a 25-inch minimum distance between the back of the guardrail post and the top of a 1V:2H or steeper slope was the standard post length increased by 12 inches? (617.03) |  |
|  | 8. Where an impenetrable object was encountered, was one of the following used: (a) a short post with a concrete anchor, (b) decreased post spacing or, (c) nesting of two rail elements as approved by the CO? (617.03) |  |
|  | 9. Post lengths or spacing was not changed in terminal sections? (617.03) |  |
|  | Rail Elements |  |
|  | 10. Were the rail elements installed after the pavement adjacent to the guardrail was installed? (617.04) |  |
|  | 11. Rail hole diameters or slot dimensions were not modified? (617.04) |  |
|  | Rail Elements (steel rail) |  |
|  | 12. Shop bent all rail elements with a radius of 150 feet or less? (617.04a) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 13. Were the rail elements erected in a smooth continuous line with the laps in the direction of traffic flow? (617.04a) |  |
|  | 14. Were bolts used that extended at least ¼ inch but not more than 1 inch beyond the nuts? (617.04a) |  |
|  | 15. Were all bolts tightened? (617.04a) |  |
|  | 16. Were all scrapes on galvanized surfaces that were through to the base metal painted with 2 coats of zinc-oxide? (617.04a) |  |
|  | Rail Elements (steel-backed timber rail) |  |
|  | 17. Were bolts along the front face of the timber rail equally spaced to match the holes in the steel backing? (617.04b) |  |
|  | 18. Were timber guardrail aligned along the top and front of the timber rail? (617.04b) |  |
|  | 19. Were timber rails field cut to produce a close fit at joints? (617.04b) |  |
|  | 20. Were field cuts treated with 2 coats of chromated copper arsenate? (617.04b) |  |
|  | 21. When required, were holes field drilled in the steel backing on curved sections to correspond to the field cut wood rails at the joints? (617.04b) |  |
|  | 22. Holes were not allowed to be cut with a torch? (617.04b) |  |
|  | Rail Elements (log rail) |  |
|  | 23. Were log rails constructed according to the plans? (617.04c) |  |
|  | Terminal Sections |  |
|  | 24. Were terminal sections constructed at the locations shown? (617.05) |  |
|  | 25. If concrete anchors were installed, were they either cast-in-place or precast units? (617.05) |  |
|  | 26. Was guardrail connected to cast-in-place anchors before the concrete cured 7 days? (617.05) |  |
|  | 27. Were end anchor cables installed tightly without slack? (617.05) |  |
|  | 28. If flared or tangent terminals were required, were drawings submitted from the manufacturer for the terminals according to Subsection 104.03? (617.05) |  |
|  | 29. If required by the contract, were earth berms constructed according to Section 204? (617.05) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Connection to Structure |  |
|  | 30. If required was the connection to a structure constructed in accordance with the plans? (617.06) |  |
|  | Removing and Resetting Guardrail |  |
|  | 31. Were posts removed and disposed of that were set in concrete? (617.07) |  |
|  | 32. Were all guardrail, posts, and hardware damaged during removal, storage, and resetting replaced? (617.07) |  |
|  | 33. Were all holes resulting from the removal of guardrail posts and anchors backfilled with granular material according to Section 209? (617.07) |  |
|  | Raising Guardrail |  |
|  | 34. Was all guardrail elements and appurtenances removed before raising? (617.08) |  |
|  | 35. Were posts reset or replaced as needed? (617.08) |  |
|  | 36. Was all guardrail, posts, and hardware damaged during the removal and raising replaced? (617.08) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: