US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 618 Concrete Barriers and Precast Guardwalls

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1A1. Was excavation and backfill performed according to Section 209? (618.03) |  |
|  | 1A2. Were barriers and guardwalls constructed to meet crash test requirements of NCHRP Report 350, Recommended Procedures for Safety Performance and Evaluation of the Highway Features? (618.03) |  |
|  | Concrete Barriers |  |
|  | 1. Were sides and top of the barrier finished according to Subsection 552.16(a)? (618.04) |  |
|  | Concrete Barriers (Cast-in-place) |  |
|  | 2. Were contraction joints hand formed or sawn ¼ in wide and 2 inches deep at 20-foot intervals? (618.04a) |  |
|  | 3. Were saw cuts done as soon as possible after the concrete had set sufficiently to preclude raveling during sawing, but before shrinkage cracking occurred? (618.04a) |  |
|  | 4. Was the saw cut depth decreased at the edge adjacent to the pavement to prevent pavement damage? (618.04a) |  |
|  | 5. Was ¾-inch preformed joint filler placed in all construction joints? (618.04a) |  |
|  | 6. Was the joint filler cut to fit the cross-sectional area at structures and barrier construction joints? (618.04a) |  |
|  | 7. Were the joint edges tool constructed? (618.04a) |  |
|  | 8. Were joints sealed according to Subsection 501.11? (618.04a) |  |
|  | Concrete Barriers (Slip-formed) |  |
|  | 9. The barrier extruded concrete surface was only touched as it left the slip-form machine to immediately remove offsets and fins by light troweling? (618.04b) |  |
|  | 10. Were adjustments made in the operation to correct any condition causing surface blemishes larger than ½ inch? (618.04b) |  |
|  | 11. Were imperfections in the completed barrier corrected without water? (618.04b) |  |
|  | Concrete Barriers (Precast) |  |
|  | 12. Were barriers precast in section lengths? (618.04c) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 13. Was the barrier foundation prepared so it did not vary over ¼ inch when a 10-foot straightedge was laid along the centerline of the barrier? (618.04c) |  |
|  | 14. Were joints aligned and connected to adjacent sections? (618.04c) |  |
|  | 15. Were cast-in-place barriers used where transitions, split barriers, or gaps shorter than 10 feet required it? (618.04c) |  |
|  | 16. At each joint between precast and cast-in-place barrier was hardware provided in the cast-in-place section to tie its end to the abutting precast section? (618.04c) |  |
|  | Precast Concrete Guardwall (Fabrication) |  |
|  | 17. Was a full-size sample of the guardwall provided at a specified location? (618.05a) |  |
|  | 18. Was the guardwall fabricated to match the sample’s shape, color, and texture? (618.05a) |  |
|  | 19. Was the precast guardwall fabricated in a concrete production facility certified by the National Precast Concrete Association and according to the Association’s Manual of Quality Control? (618.05a1) |  |
|  | 20. Were the facing mixes, backing mixes, and structural concrete backup formulated to produce concrete mix designs of similar aggregate-cement ratios to minimize differences in shrinkage factors and coefficients of thermal expansion and contraction? (618.05a2) |  |
|  | 21. Was the concrete formulated using hydraulic cement, limestone, quartz, mica, and silicious stones in such proportions as to match the sample? (618.05a2) |  |
|  | 22. Was epoxy coated reinforcing steel used at locations where the reinforcing steel was less than 2 inches from the exposed surface? (618.05a3) |  |
|  | 23. Were the segments cast straight and true to a line in strong permanent composite molds of steel, plastic resins, concrete, or rubber? (618.05a4) |  |
|  | 24. Were the facing mixes cast a minimum of 1 inch thick? (618.05a5) |  |
|  | 25. Was a good bond between facing and backup mixes assured? (618.05a5) |  |
|  | 26. Were 4 lifting inserts provided in unexposed areas? (618.05a6) |  |
|  | 27. Were removable caps provided for the lifting inserts to allow for future segment replacement? (618.05a6) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Precast Concrete Guardwall (Test section) |  |
|  | 28A. Was the ability to match the sample demonstrated by fabricating a 10-foot, full-scale guardwall test section and delivering it to the location of the sample for comparison? (618.05b) |  |
|  | 28B. If the test section did not match the original full size sample with reasonably close conformity was another one fabricated according to Subsection 618.05 (1) thru (6)? (618.05b) |  |
|  | 29. Rejected test sections were not used in the wall installation? (618.05b) |  |
|  | Precast Concrete Guardwall (Installation) |  |
|  | 30. After the test section was approved, were the rest of the guardwall sections produced to match the approved text section? (618.05c) |  |
|  | 31. Were the segments not damaged during fabrication, handling, delivery or installation? (618.05c) |  |
|  | 32. Were all damaged section repaired or replaced? (618.05c) |  |
|  | 33. Was the foundation prepared before placing the sections? (618.05c) |  |
|  | 34. Were backer rods and joint sealant used in the section joints to match the false joints? (618.05c) |  |
|  | 35. At 100-foot intervals and at low points in the guardwall, were outlet ditches dug and filled with 6 inches of aggregate conforming to AASHTO M 43, number 57? (618.05c) |  |
|  | Terminal Sections |  |
|  | 36. If barrier is being constructed next to roadway lanes open to traffic, was an approved temporary section connected to the barrier at the end of each day? (618.06) |  |
|  | 37. Were permanent graded berms constructed according to Section 204? (618.06) |  |
|  | Resetting Barrier |  |
|  | 38. Were barrier and terminal sections reset according to Subsections 618.03 and 618.06? (618.07) |  |
|  | 39. Were barrier sections stored in an approved location when resetting could not immediately follow removal? (618.07) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: