US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 619 Fences, Gates, and Cattle Guards

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |
| --- | --- |
| Yes No N.A. |  |
|  | Fences and Gates (General) |
|  | 1A. Was the fence line cleared? (619.03) |
|  | 1. Were trees, brush, logs, upturned stumps, roots of downed trees, rubbish, and debris removed and disposed of according to Subsection 201.06? (619.03a) |
|  | 2. Was a 10-foot width for chain link fence or a 3-foot width for wire fence cleared? (619.03a) |
|  | 3. Was grubbing only completed where short and abrupt changes in the ground contour require the removal of stumps to properly grade the fence line? (619.03a) |
|  | 4. Were stumps within the clearing limits removed or close cut? (619.03a) |
|  | 5. Was clearing and leveling performed with minimum disturbance to the terrain outside the fence line? (619.03a) |
|  | 6. Was the fence installation, temporary fence, or other adequate means to prevent livestock from entering the project right-of-way, easements or adjoining properties scheduled? (619.03a) |
|  | 7. Were new fences connected to bridges, cattle underpasses, and culverts to permit free passage of livestock under or through the structure? (619.03a) |
|  | 8. Were chain link fences and gates constructed according to Subsections 619.03(b1-5)? (619.03b) |
|  | 9A. Were wire fences and gates constructed according to Subsection 619.03(c1-5)? (619.03c) |
|  | Grounding Fences |
|  | 9B. Were fences grounded according to Subsection 619.04? (619.04) |
|  | Remove and Reset Fence |
|  | 9C. Were fences removed and reset according to Subsection 619.05? (619.05) |
|  | Temporary Fence |
|  | 9D. Were temporary fences installed according to Subsection 619.06? (619.06) |
|  | Cattle Guards (Excavating and backfilling) |
|  | 10. Was the work performed as described under Section 209? (619.07a) |
|  | 11. Was foundation excavated to depth with sufficient space for proper installation of formwork? (619.07a) |
|  | 12. If cattle guard was installed on new embankment, was the embankment completed and compacted according to Section 204 before excavating for footing? (619.07a) |
| Conformance | CHECKS (characteristics) |
| Yes No N.A. |  |
|  | Cattle Guards (Concrete foundation) |
|  | 13. Were cast-in-place concrete foundations constructed according to Section 601? (619.07b) |
|  | 14. Were precast units set in the foundation concrete before it hardened? (619.07b) |
|  | 15. Were the stringer bearings finished to allow full bearing under each stringer? (619.07b) |
|  | 16. Did the cattle guard rest on the concrete without rocking? (619.07b) |
|  | Cattle Guards (Cattle guard) |
|  | 17. Was cattle guard fabricated according to Section 555? (619.07c) |
|  | 18. Were guards assembled and placed as shown on the plans? (619.07c) |
|  | 19. Was the cattle guard securely fastened to the foundation? (619.07c) |
|  | 20. Were the metal wings fastened to the cattle guard as shown on the plans? (619.07c) |
|  | 21. Were fences and gates connected according to the plans? (619.07c) |
|  | 22. Was welding performed according to ANSI/AASHTO/AWS D1.5? (619.07c) |
|  | 23. If standard manufactured cattle guards were used were they approved? (619.07c) |
|  | 24. Did designs provide for AASHTO loading M-18? (619.07c) |
|  | 25. Were suitable cleanouts provided? (619.07c) |
|  | 26. Were drawings prepared and submitted according to Subsection 104.03? (619.07c) |
|  | Cattle Guards (Painting) |
|  | 27. Did all metal parts receive one shop coat? (619.07d) |
|  | 28. Did two additional coats of paint get applied in the shop or in the field? (619.0d) |
|  | 29. Was paint according to Section 563? (619.07d) |

|  | Bollards |  |
| --- | --- | --- |
|  | 30. Were holes drilled for Bollards? (619.08) |  |
|  | 31. Were posts set plumb, and backfilled with approved material and compacted? (619.08) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: