US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 620 Stone Masonry

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. Was stone that matches the native stone on the project furnished? (620.03) |  |
|  | 2. Were stone samples submitted that represented the range of colors and sizes to be used on the project to the CO 14 days before beginning work? (620.03) |  |
|  | 3. Was an adequate inventory of stone kept on site to provide an ample variety of stones for the masons? (620.03) |  |
|  | 4. If additional stone was added, was the new stone mixed with the existing stone in a uniform pattern and color? (620.03) |  |
|  | 5. Was excavation and backfill performed according to Section 209? (620.03) |  |
|  | 6. Was the foundation bed prepared normal to, or in steps normal to, the face of the masonry? (620.03) |  |
|  | 7. Where foundation masonry was used, was the bearing surface cleaned thoroughly and wet immediately before spreading the mortar bed? (620.03) |  |
|  | Placing Stone |  |
|  | 8. Was stone placed to provide a uniform pattern and color? (620.04) |  |
|  | 9. Was stone masonry placed when the ambient temperature was above 32°F? (620.04) |  |
|  | 10. Was completed masonry maintained at a temperature above 40°F for 24 hours after construction? (620.04) |  |
|  | 11. Were all stones cleaned thoroughly and moistened immediately before placing? (620.04) |  |
|  | 12. Was the bed cleaned and moistened? (620.04) |  |
|  | 13. When removing and resetting stone masonry, were hand tools used to clean the exposed faces of the stones of all mortar before resetting? (620.04) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 14. When spreading the mortar the thicknesses of beds and joints for face stones are shown in Table 620-1. Did the bed and joint thicknesses meet the table specifications? (620.04) |  |
|  | 15. Were stone joints on the faces and soffits ringed not less than ¼ inch or more than 1 ½ inches thick? (620.04) |  |
|  | 16. Was the bed of each course of uniform thickness throughout? (620.04) |  |
|  | 17. Were joints in dimensioned masonry constructed vertical? (620.04) |  |
|  | 18. If vertical walls were cross beds level? (620.04) |  |
|  | 19. Were stones with the longest face laid horizontal and the exposed face parallel to the masonry face? (620.04) |  |
|  | 20. Were the joints flush with mortar? (620.04) |  |
|  | 21. Were stones already set jarred or displaced? (620.04) |  |
|  | 22. If a stone was loosened after the mortar had taken initial set was it removed, cleaned off and re-laid with fresh mortar? (620.04) |  |
|  | Pointing |  |
|  | 22A. Was pointing performed according to Subsection 620.06? (620.06) |  |
|  | Constructing Walls |  |
|  | 23B. Was an L-shaped sample section of wall not less than 5 feet high and 8 feet long, showing examples of face wall, top wall, method of turning corners, and method of forming joints constructed? (620.06) |  |
|  | 24. Was only foundation masonry constructed before the sample was approved? (620.06) |  |
|  | 25. Were face stones set in a random bond to produce the effect shown on the plans and to correspond with the approved sample section? (620.06) |  |
|  | 26. Beds were not extended in an unbroken line through more than 5 stones and joints through more than 2 stones? (620.06) |  |
|  | 27. Was each face stone bonded with all contiguous face stones at least 6 inches longitudinally and 2 inches vertically? (620.06) |  |
|  | 28. Was construction such that the corners of four stones were not adjacent to each other? (620.06) |  |
|  | 29. Small stones or stones of the same size, color, or texture were not bunched together? (620.06) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 30. In general do the stones decrease in size from the bottom to the top of the work? (620.06) |  |
|  | 31. Were large stones used for the bottom courses and large selected stones used in corners? (620.06) |  |
|  | 32. Where required, were headers distributed uniformly throughout the walls of structures to from at least 20 percent of the faces? (620.06a) |  |
|  | 33. Was backing constructed out of large stones? (620.06b) |  |
|  | 34. Were the individual stones composing the backing and heart bonded with the stones in the face wall and with each other? (620.06b) |  |
|  | 35. Were all openings and interstices filled in the backing completely with mortar or with spalls surrounded completely by mortar? (620.06b) |  |
|  | 36. Were copings constructed as shown on the plans? (620.06c) |  |
|  | 37. Where copings were not called for, was the top of the wall finished with stones wide enough to cover the top of the wall from 1 ½ to 5 feet in length, and of random heights, with a minimum height of 6 inches? (620.06c) |  |
|  | 38. Were stones laid in a manner that the top course was an integral part of the wall? (620.06c) |  |
|  | 39. Were the tops of the top courses of stone pitched to line in both vertical and horizontal planes? (620.06c) |  |
|  | 40. Were selected stone used, and were they squared and pitched to line and with heads dressed in the ends of parapet walls and in all exposed angles and corners? (620.06d) |  |
|  | 41. Were headers interlocked with as many headers as possible extending entirely through the wall? (620.06d) |  |
|  | 42. Did the headers and stretchers comprise practically the whole volume of the wall? (620.06d) |  |
|  | 43. Were all interstices and spalls completely filled with mortar? (620.06d) |  |
|  | 44. Were weep holes provided for all walls and abutments? (620.06e) |  |
|  | 45. Were weep holes placed at the lowest points where free outlets could be obtained and spaced no more than 10 feet center to center? (620.06e) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Facing for Concrete (Stone placed before concrete) |  |
|  | 46. Was the back of the masonry uneven to improve the bond to the concrete backing? (620.06a) |  |
|  | 47. Was No.4 reinforcing steel bent into an elongated letter S used to anchor the stone? (620.06a) |  |
|  | 48. Was each anchor embedded in a mortar bed to within 2 inches from the face of the stones? (620.06a) |  |
|  | 49. Was the other end projected ± 10 inches into the concrete backing? (620.06a) |  |
|  | 50. Were the anchors spaced 18 inches apart both horizontally and vertically? (620.06a) |  |
|  | 51. After the mortar attained sufficient strength, was the back masonry surface cleaned of all dirt, loose material, and mortar drippings? (620.06a) |  |
|  | 52. Were surfaces washed just before placing the concrete using a high-pressure water jet? (620.06a) |  |
|  | 53. When placing the concrete, was a neat cement grout of the consistency of cream on top of the concrete and against the masonry at all times? (620.06a) |  |
|  | 54. Were all interstices coated in the back of the masonry with grout? (620.06a) |  |
|  | Facing for Concrete (Concrete placed before stone) |  |
|  | 55. Was the facing thickness as shown on the plans? (620.07b) |  |
|  | 56. Were galvanized metal slots set with anchors in the concrete face? (620.07b) |  |
|  | 57. Were the anchors set vertically at a horizontal spacing not exceeding 24 inches? (620.07b) |  |
|  | 58. Were temporary fillings of felt or other material placed in the slots to prevent filling with concrete? (620.07b) |  |
|  | 59. Where setting the stone facing, were the metal anchors fit tightly in the slots at an average vertical spacing of 24 inches? (620.07b) |  |
|  | 60. Were at least 25 percent of the anchors bent at a short right angle to engage a recess cut in the stone? (620.07b) |  |
|  | 61. Were the anchors extended to within 3 inches of the exposed face of the stone work? (620.07b) |  |
|  | 62. Where the shape of the concrete face was unsuitable for the use of metal slots, were 9 gage galvanized iron wire ties used at a rate of 6 ties for each square yard of exposed surface? (620.07b) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 63. Were the ties installed after the concrete had cured, using a gun? (620.07b) |  |
|  | 64. Were the concrete faces continuously kept wet for 2 hours preceding the placing of the stone and were spaces between the stones and concrete filled with mortar? (620.07b) |  |
|  | Constructing Arches |  |
|  | 65. Were drawings prepared and submitted for falsework according to Section 562? (620.08) |  |
|  | 66. Were arch ring stones stratified parallel to the radial joint and other stones stratified parallel to the beds? (620.08) |  |
|  | 67. Was a full-size template of the arch ring laid out near the quarry site showing face dimensions of each ring stone and thickness of joints? (620.08) |  |
|  | 68. Was approval received before the shaping of any ring stone was started and were any ring stone placed in the structure before all ring stones had been shaped and dressed? (620.08) |  |
|  | 69. Was arch centering constructed according to the approved drawings? (620.08) |  |
|  | 70. Were suitable wedges provided for adjusting the elevation of the forms? (620.08) |  |
|  | 71. Were arch ring stones set to the exact position and held in place with hardwood wedges until the joints were packed with mortar? (620.08) |  |
|  | 72. When required, was the centering supported with approved jacks to correct settlement after masonry placement began? (620.08) |  |
|  | 73. Was the centering lowered gradually and symmetrically to avoid overstresses in the arch? (620.08) |  |
|  | 74. Was the arch self-supporting before the railing or coping was placed? (620.08) |  |
|  | 75. For filled spandrel arches, were the centers struck before constructing the spandrel walls to avoid jamming of the expansion joints? (620.08) |  |
|  | 76. Was the backfill placed so the ring was uniformly and symmetrically loaded? (620.08) |  |
|  | Guardwall |  |
|  | 77. Was rubble masonry used? (620.09) |  |
|  | 78. Were concrete corewalls for guardwall cast-in-place or precast units according to Section 601? (620.09) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 79. Did concrete have a minimum 28-day compressive strength of 3,500 pounds per square inch? (620.09) |  |
|  | 80. Was a 25-foot sample section of guardwall constructed? (620.09) |  |
|  | 81. Additional guardwall was not constructed until the sample section was approved? (620.09) |  |
|  | 82. Was the guardwall constructed true and uniform along its length with no stone projecting more than 1½ inches? (620.09) |  |
|  | 83. Were mortar beds and joints made according to Table 620-1? (620.09) |  |
|  | 84. Were the joints and bed raked to a depth of 2 inches on the front and top sides and to 1½ inches on the back? (620.09) |  |
|  | 85. Was a one-piece capstone used the full width of the guardwall for at least 25 percent of the total length? (620.09) |  |
|  | 86. Was a two-piece capstone with the joint within 4 inches of the guardwall center used for the remaining length? (620.09) |  |
|  | 87. Were all stones, including the capstones, placed randomly to avoid a pattern? (620.09) |  |
|  | 88. Were stones laid to reflect the width of the expansion joints? (620.09) |  |
|  | 89. There were no gap or mortar edges left at the expansion joint? (620.09) |  |
|  | 90. Were various sized stones used to coin or key the corners of the guardwall? (620.09) |  |
|  | 91. |  |
|  | 92. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: