US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 624 Topsoil

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Preparing Areas |  |
|  | 1. Were all slopes and disturbed areas to be covered with topsoil shaped? (624.03) |  |
|  | 2. Were all slopes 1V:3H or flatter disked or scarified to a depth of 4 inches? (624.03) |  |
|  | Placing Topsoil |  |
|  | 3. Was at least 7 days notice provided before the start of topsoil placement? (624.04) |  |
|  | 4. Topsoil was not placed when the ground or topsoil was frozen, excessively wet, or otherwise in a condition detrimental to the work? (624.04) |  |
|  | 5. Was all conserved topsoil used before furnishing topsoil? (624.04) |  |
|  | 6. Was the roadway surfaces kept clean during hauling and spreading operations? (624.04) |  |
|  | 7. Was the topsoil spread to a depth that, after settlement, provided the required depth? (624.04) |  |
|  | 8. Were clods and lumps broken up with harrows, disks, or other appropriate equipment to provide a uniform textured soil? (624.04) |  |
|  | 9. Were clods and stones larger than 2 inches, stumps, roots, and other litter removed and disposed of according to Subsection 203.05? (624.04) |  |
|  | 10. Were longitudinal depressions constructed at least 2 inches deep perpendicular to the natural flow of water without overly compacting the topsoil surface? (624.04) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: