US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 625 Turf Establishment

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1A1. Was turf establishment applied to finished slopes with in 14 days of completion of construction on that portion of the site? (625.03) |  |
|  | 1A2. Seeding did not occur during windy weather or when the ground is excessively wet, frozen, snow covered, extremely dry, cloddy, hard pan, or not friable? (625.03) |  |
|  | Preparing Seedbed |  |
|  | 1. Was the seeding area graded to line and grade? (625.04) |  |
|  | 2. Were all weeds, sticks, stones 2 inches in diameter and larger and other debris detrimental to application, growth, or maintenance of the turf removed? (625.04) |  |
|  | 3. If required by the contract was limestone and grubproofing applied? (625.04) |  |
|  | 4. Was the seeding area cultivated to a minimum depth of 4 inches and a firm but friable seedbed prepared before seeding? (625.04) |  |
|  | Watering |  |
|  | 5. Was seeding area moistened before seeding and maintained moist for 10 days after germination? (625.05) |  |
|  | Fertilizing |  |
|  | 6. If fertilizing via the dry method, was the fertilizer incorporated into the upper portion of the seedbed before seeding? (625.06) |  |
|  | 7. If fertilizing via the hydraulic method, was fertilizer added to the slurry and mixed before adding seed? (625.06) |  |
|  | 8. If fertilizing via the hydraulic method, was the seed and fertilizer applied in one application? (625.06) |  |
|  | Seeding (Dry method) |  |
|  | 9. Was the seed applied with an approved power driven seeder, drill, or other mechanical equipment? (625.07a) |  |
|  | 10. Was a hand-operated seeding method used on areas inaccessible to mechanical equipment? (625.07a) |  |
|  | 11. Was the seedbed lightly compacted within 24 hours after seeding? (625.07a) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Seeding (Hydraulic method) |  |
|  | 12. Was hydraulic-type equipment used that was capable of providing a uniform application using water as the carrying agent? (625.07b) |  |
|  | 13. Was a tracer material added to the water that consisted of either wood or grass cellulose fiber mulch? (625.07b) |  |
|  | 14. Was the tracer material applied at a rate of 400 pounds per acre to provide visible evidence of uniform application? (625.07b) |  |
|  | 15. Was the seed not added to the water slurry more than 30 minutes before application? (625.07b) |  |
|  | 16. Were inaccessible areas seeded by hand held seeding equipment? (625.07b) |  |
|  | Mulching (Dry method) |  |
|  | 17. Was all mulch material, except wood and grass cellulose fibers, spread by a mulch spreader utilizing forced air to blow the mulch material onto the seeded area? (625.08a) |  |
|  | 18. Was straw mulch applied at a rate of 3200 pounds per acre? (625.08a) |  |
|  | 19. Was the mulch material anchored with an approved stabilizing emulsion tackifier or approved mechanical method? (625.08a) |  |
|  | 20. Structures, pavements, utilities or plant growth were not marked or defaced with tackifier?(625.08a) |  |
|  | Mulching (Hydraulic method) |  |
|  | 21. Was mulch applied in a separate application from the seed using hydraulic-type equipment according to Subsection 625.07(b)? (625.08b) |  |
|  | 22. Was wood fiber or grass straw cellulose fiber mulch applied at a rate of 1500 pounds per acre? (625.08b) |  |
|  | 23. Was bonded fiber matrix hydraulic mulch applied at a minimum rate of 3000 pounds per acre? (625.08b) |  |
|  | 24. Was the mulch applied so there were no holes greater than 0.04 inches left in the matrix? (625.08b) |  |
|  | 25. Was the mulch applied so there were no gaps existing between the matrix and the soil? (625.08b) |  |
|  | 26. Were areas inaccessible to mulching equipment mulched by hand? (625.08b) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: